VCDS COMPLETE REFERENCE — EXHAUSTIVE EDITION

2018 Audi A3 8V MQB — EA888 Gen3 CZPB

EVERY Basic Settings Routine | EVERY Adaptation Channel | EVERY Live Data Channel — All Modules

# MODULE 01 — All Basic Settings Routines

Security Access: 27971 (general). No code required for most calibrations. Engine running unless noted.

| Routine Name (search keyword) | Prerequisites | Procedure & Expected Outcome |
| --- | --- | --- |
| Throttle Body Adaptation (TBA) 'Throttle valve adaptation' | Engine OFF. Ignition ON. Engine WARM (90°C). No active fault codes. | Engine off ignition on only. Click Do It. Throttle cycles open/closed audibly. ADP OK = complete. Hear throttle body motor cycle. Duration: 10–30s. Post-check: IDE01377 shows 2–5°. |
| Idle Speed Adaptation 'Idle speed adaptation' | Engine RUNNING. Fully warm (90°C). All electrical loads OFF. Vehicle stationary. | Runs 30–90 seconds at idle. RPM may fluctuate ±50 RPM. ADP OK when complete. Result: stable warm idle at 620–720 RPM. |
| Camshaft/Timing Chain Adaptation 'Camshaft adaptation' | Engine running warm. After any timing chain or cam work. | Runs 30–120 seconds at idle. ECM samples cam-crank relationship. After: IDE00182 and IDE00184 should read near 0° on fresh chain. |
| Injector Adaptation (IMA) 'Injector matching' | Ignition ON. Security Access 12233 or 79153. New injector IMA code from injector label. | Enter 6-character IMA code per cylinder via Adaptation channel (not Basic Settings). Then clear codes and verify smooth idle. |
| IMRC Adaptation 'Intake manifold runner adaptation' | Engine running warm. After IMRC sensor/manifold replacement or P2015. | Flaps cycle full range (15–30 seconds). ADP OK. Post-check: IDE00401 matches IDE00400 across load range. |
| Fuel Trim Reset 'Fuel trim reset' or 'Mixture adaptation reset' | Can be cold or warm. After injector, HPFP, or O2 sensor work. | Completes in 2–5 seconds. IDE00028 resets to 0%. Drive 50+ miles mixed for relearn. Engine may feel different for 10–30 miles — normal. |
| O2 Sensor Adaptation Reset 'Lambda sensor adaptation reset' | Engine warm. After replacing upstream or downstream O2 sensor. | Clears stored O2 heater and response corrections. Drive two full warm-up cycles for relearn. |
| Knock Sensor Self-Test 'Knock sensor test' | Engine running warm. | ECM generates test signal and verifies sensor response. IDE01098 shows Pass or Fail. Fail with correctly installed sensor = faulty sensor or wiring. |
| EVAP Leak Test 'Tank leak diagnosis' | Engine warm. Fuel level 15–85% (not full or very empty). Vehicle stationary. | Closes vent valve. Monitors pressure change for 2–3 minutes. Reports gross leak / small leak / no leak. Confirm via IDE00424 after. |
| Cylinder Contribution Test 'Cylinder balance' | Engine warm at idle. No active misfire codes. | Cuts fuel to each cylinder individually. Measures RPM drop. Healthy cylinder: 80–150 RPM drop. Weak: less than 40 RPM drop. |
| Relative Compression Test 'Cranking compression' | Engine warm. Battery fully charged. Ignition ON engine OFF. | Disables injection. Crank engine 3–5 seconds. VCDS analyzes crankshaft speed variation. Reports relative compression per cylinder as percentage. |
| VVT Solenoid Test 'Variable valve timing test' | Engine warm. | Activates each VVT solenoid and measures cam response time. IDE00902/903 response times verified. Pass = solenoid and oil pressure adequate. |
| Boost Leak Test (if supported) 'Boost system test' | Engine warm. Can be stationary. | Closes intake system and monitors pressure decay. Identifies boost leaks by pressure drop rate. Not present on all Simos 18.1 software versions. |
| Fuel System Pressure Test 'Fuel system test' | Engine warm. | Commands HPFP to maximum, then minimum pressure. Monitors IDE00200 response. Verifies pump and pressure regulator function. |
| Catalytic Converter Test 'Catalyst test' | Engine warm. O2 sensors at operating temp. | Monitors pre- and post-cat O2 sensors during specific load cycle. Reports catalyst efficiency. Pass = IDE01644 stable 0.6–0.9V. |
| EGR System Test (if equipped) 'EGR test' | Engine warm. | Opens and closes EGR valve while monitoring MAF and idle. Not present on all CZPB variants. |
| Secondary Air Test (if equipped) 'Secondary air test' | Cold engine preferred (below 30°C coolant). First start of day. | Activates air pump on cold start. Monitors O2 sensor response to secondary air injection. Not on all variants. |
| Oil Level Check 'Oil level check' | Engine off. Level surface. Wait 3 minutes after shutoff. | Reads oil level sensor (if equipped). Displays level in mm or relative level. Supplement with physical dipstick check. |
| Cooling Fan Test 'Cooling fan' | Engine running or ignition ON. | Activates cooling fan at stage 1 and stage 2. Verifies relay and motor function. Fan should change speed audibly between stages. |
| Engine Speed Governor Test 'Speed governor' | Engine running. | Verifies speed limiter function. Reports governed maximum speed value stored in ECM. |

# MODULE 01 — All Adaptation Channels

Security Access: 27971. Access via Module 01 → [Adaptation] → search dropdown.

| Channel Name (search keyword) | Read Value Meaning | Reset/Write Value | When to Modify |
| --- | --- | --- | --- |
| Long-term fuel trim bank 1 'mixture correction' | Current learned fuel correction. Positive = ECU adding fuel (lean). Negative = reducing (rich). Range: -25% to +25%. | Set to 0 to reset. ECM relearns over 50+ miles. | After injector replacement, O2 sensor replacement, MAF replacement, or after resolving a vacuum leak. |
| Additive mixture correction bank 1 'additive mixture' | Per-injection-event timing correction in ms. Reflects injector offset from nominal. | Set to 0 to reset. | After injector replacement. Enters relearn automatically — usually no manual change needed. |
| Multiplicative mixture correction bank 1 'multiplicative mixture' | Proportional fuel correction factor %. Should be near 0%. | Set to 0 to reset. | Rarely modified manually. Resets with fuel trim reset routine. |
| Idle speed offset 'idle speed' | Learned RPM offset to maintain target idle. Normal: ±50 counts. | Set to 0 to reset idle learning. | After throttle body cleaning, intake work, or when idle is unstable after repair. |
| Knock correction cylinder 1 'knock cyl 1' | Stored timing retard for cylinder 1. 0 = no knock detected. | Set to 0 to reset. | After knock sensor replacement. Persistent non-zero on premium fuel = investigate. |
| Knock correction cylinder 2 'knock cyl 2' | Same for cylinder 2. | Set to 0. | Same conditions as cylinder 1. |
| Knock correction cylinder 3 'knock cyl 3' | Same for cylinder 3. | Set to 0. | Same conditions. |
| Knock correction cylinder 4 'knock cyl 4' | Same for cylinder 4. | Set to 0. | Same conditions. |
| VVT intake cam offset 'camshaft adaptation intake' | Timing chain wear indicator. New = near 0°. Warning = ±4°. Critical = ±6°. | Set to 0 ONLY after fresh timing chain installation and running camshaft adaptation Basic Settings. | After timing chain replacement. NEVER reset without physical chain work. |
| VVT exhaust cam offset 'camshaft adaptation exhaust' | Same function for exhaust cam/AVS. | Set to 0 only after chain work + Basic Settings routine. | Same as intake offset. |
| Injector IMA code cylinder 1 'injector adaptation cyl 1' | 6-character calibration code for cylinder 1 GDI injector. | Enter exact IMA code from injector label. | After replacing cylinder 1 GDI injector. |
| Injector IMA code cylinder 2 'injector adaptation cyl 2' | Same for cylinder 2. | Enter IMA code. | After cylinder 2 GDI replacement. |
| Injector IMA code cylinder 3 'injector adaptation cyl 3' | Same for cylinder 3. | Enter IMA code. | After cylinder 3 replacement. |
| Injector IMA code cylinder 4 'injector adaptation cyl 4' | Same for cylinder 4. | Enter IMA code. | After cylinder 4 replacement. |
| O2 sensor heater correction 'oxygen sensor heater' | Learned heater power correction for upstream O2 sensor. Ensures sensor reaches operating temp in specified time. | Set to 0 after O2 sensor replacement. | After replacing upstream wideband O2 sensor. |
| HPFP adaptation 'fuel pump adaptation' | High-pressure fuel pump learned delivery correction. | Set to 0 after HPFP replacement. | After replacing high-pressure fuel pump. |
| IMRC closed position offset 'IMRC offset' | Learned voltage reference for IMRC fully-closed position. | Relearned automatically after IMRC adaptation Basic Settings. | After IMRC sensor or manifold replacement. |
| Throttle body lower stop 'throttle lower stop' | Stored mechanical fully-closed position from TBA. | Relearned via TBA Basic Settings — do not modify manually. | Read-only reference. Reset via TBA routine. |
| Throttle body upper stop 'throttle upper stop' | Stored mechanical fully-open position. | Same as lower stop. | Read-only reference. |
| Fuel tank capacity 'tank capacity' | Stored usable fuel volume. Used for range calculations. | Enter actual tank capacity in liters if replacing fuel tank. | After fuel tank replacement with different capacity. |
| Combustion mode diagnostic 'combustion mode' | Forces GDI-only or MPI-only injection for diagnostic testing. | Active / Inactive. Return to Inactive after testing. | Diagnostic use only to isolate injector system faults. |
| Start-stop: engine function 'start-stop engine' | Whether start-stop is enabled at ECM level. | Active / Inactive. | Permanent start-stop disable (prefer Module 61 BMS approach for better battery management). |
| Exhaust flap: deactivation 'exhaust flap' | Whether exhaust valve control is active (sport exhaust equipped cars). | Active / Inactive. | Permanently hold exhaust flaps open for louder exhaust note. |
| Engine speed limit 'speed limiter' | Maximum engine RPM. | Factory: 6500 RPM. Adjustable within safe range. | Do not modify unless advised by specific tuning documentation. |

# MODULE 01 — Complete Live Data Channel Reference

These supplement the primary IDE channel reference. Organized by system for easy navigation.

## Engine Speed & Load — Complete

| IDE Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| IDE00001 | Engine speed (RPM) | /min | Idle warm: 620–720. Cold idle: 900–1100. |
| IDE00002 | Engine speed: specified (target) | /min | ECU target idle RPM. Actual should match within 50 RPM. |
| IDE00003 | Engine speed: maximum allowed | /min | Rev limiter setting. Typically 6500 RPM. |
| IDE00014 | Engine load: calculated | % | Idle: 5–15. Cruise: 25–55. WOT: 80–100. |
| IDE00015 | Engine load: specified | % | ECU target load. Should track actual. |
| IDE00349 | Throttle valve: normed position | % | 0% idle, 100% WOT. |
| IDE01377 | Throttle valve adapter: actual | ° | 2–5° after fresh TBA. |
| IDE01649 | TPS sensor 1 voltage | V | 0.65V idle, 4.3V WOT. |
| IDE01650 | TPS sensor 2 voltage | V | Always ~half of sensor 1. Ratio = 2:1. |
| IDE00392 | Accelerator pedal sensor 1 | V | 0.75V rest, 3.8V full. |
| IDE00393 | Accelerator pedal sensor 2 | V | 0.38V rest, 1.9V full. Always half of sensor 1. |
| IDE00358 | Torque: actual limitation | Nm | 220 Nm normal. Lower = ECU limiting torque due to fault or protection. |
| IDE00359 | Torque: driver request | Nm | What driver is requesting via throttle. |
| IDE00360 | Torque: maximum available | Nm | Maximum the ECU will allow at current conditions. |

## Temperature Sensors — Complete

| IDE Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| IDE00005 | Coolant temperature | °C | Normal op: 87–100. Cold start: ambient. |
| IDE00006 | Coolant temperature: specified | °C | ECU target temp. Typically 90°C. Deviation = thermostat issue. |
| IDE00007 | Intake air temperature (pre-throttle) | °C | Ambient + 5–15°C when hot. |
| IDE00008 | Ambient air temperature | °C | Outside air temp as measured at sensor. |
| IDE00012 | Oil temperature | °C | Normal: 100–115°C. |
| IDE00348 | Charge air temperature (post-intercooler) | °C | Near ambient when moving. High = intercooler issue. |
| IDE00357 | Fuel temperature | °C | 40–70°C normal. |
| IDE00500 | Cold start: enrichment active | — | Active / Inactive. Active below ~60°C coolant. |
| IDE00505 | Catalyst temperature: estimated | °C | 0°C cold start, rises to 350–800°C normal operation. |

## Fuel Pressure — Complete (High and Low Pressure Systems)

| IDE Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| IDE00200 | High fuel pressure: actual | bar | Idle: 40–60. Cruise: 70–100. WOT: 120–180. |
| IDE00201 | High fuel pressure: specified | bar | ECU target. Tracks actual. Gap >15 bar = HPFP fault. |
| IDE00202 | Low fuel pressure: specified | bar | 4.5–5.5 bar constant. Below 4 = LP pump fault. |
| IDE00203 | Low fuel pressure: actual | bar | Should match specified within 0.5 bar. |
| IDE01378 | High pressure: control deviation | bar | 0–5 bar normal. Above 10 = HPFP fault. |
| IDE00304 | Fuel level | L | 0–70L. Verify accuracy vs gauge. |
| IDE00371 | Fuel consumption: instantaneous | L/h | Idle: 0.8–1.5. Cruise: 5–9. WOT: 25–45. |
| IDE00372 | Fuel consumption: average | L/100km | Running average since last reset. |
| IDE00550 | Injector correction cylinder 1 | ms | Per-cylinder balance. ±0.3 ms normal. |
| IDE00551 | Injector correction cylinder 2 | ms | Same cyl 2. |
| IDE00552 | Injector correction cylinder 3 | ms | Same cyl 3. |
| IDE00553 | Injector correction cylinder 4 | ms | Same cyl 4. |
| IDE00554–557 | Injection quantity actual cyl 1–4 | mg/stroke | Actual delivery per cylinder. Compare to commanded. |
| IDE00558 | Injection quantity: commanded | mg/stroke | ECU target per cylinder. |

## Lambda / O2 Sensors — Complete

| IDE Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| IDE00024 | Lambda bank 1: specified | Lambda | 1.000 cruise. <1 WOT (rich). >1 decel (lean). |
| IDE00025 | Lambda bank 1: actual | Lambda | Tracks specified at cruise. Active oscillation = healthy control loop. |
| IDE00026 | Lambda bank 1: control deviation | Lambda | Difference between actual and specified. Near 0 = good control. |
| IDE00027 | Short-term fuel trim bank 1 | % | ±5% normal. Beyond ±15% = fault. |
| IDE00028 | Long-term fuel trim bank 1 | % | ±10% normal. Beyond ±20% = diagnose. |
| IDE00029 | Additive mixture correction | ms | Near 0 ms normal. |
| IDE00030 | Multiplicative mixture correction | % | Near 0% normal. |
| IDE01644 | Downstream O2 bank 1 sensor 2 voltage | V | 0.6–0.9V = healthy catalyst. |
| IDE01196 | Dynamic factor O2 sensor 1 | — | 0 at rest. Changes during diagnostic test. |
| IDE01638 | O2 sensor aging test result | — | Pass / Fail / Not started. |
| IDE01639 | O2 sensor heater: actual power | W | Power consumed by upstream O2 heater element. |
| IDE01640 | O2 sensor: response time | ms | How fast sensor transitions rich-to-lean. >250ms = aging sensor. |

## Ignition & Knock — Complete

| IDE Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| IDE00036 | Ignition timing: actual bank 1 | °BTDC | Idle: 8–18. Cruise: 20–35. WOT: 5–22. |
| IDE00037 | Knock retard cylinder 1 | ° | 0° ideal. Persistent -4° + = investigate. |
| IDE00038 | Knock retard cylinder 2 | ° | Same cyl 2. |
| IDE00039 | Knock retard cylinder 3 | ° | Same cyl 3. |
| IDE00040 | Knock retard cylinder 4 | ° | Same cyl 4. |
| IDE00041 | Knock sensor 1: signal amplitude | mV | Low amplitude at idle (0.1–0.5 mV). Higher under load. Too low = faulty sensor. |
| IDE00042 | Knock sensor 2: signal amplitude | mV | Same reference. |
| IDE00810–813 | Knock event counter cyl 1–4 | Count | Running knock events per cylinder. Reset when codes cleared. High on one cylinder = investigate that cylinder. |
| IDE00820–823 | Misfire counter cyl 1–4 | Count | Running misfire events per cylinder. Specific cylinder high count directs diagnosis. |
| IDE00824 | Misfire rate: current | % | Above 2% = active misfire fault threshold. |
| IDE01098 | Knock sensor self-test result | — | Pass / Fail. Run knock sensor Basic Settings test. |

## VVT & Timing Chain — Complete

| IDE Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| IDE00045 | Intake cam actual position | °CRK | 0–40° depending on load/RPM. Active VVT adjustment. |
| IDE00046 | Exhaust cam actual position | °CRK | 0–30° depending on load/RPM. |
| IDE00047 | Intake cam specified position | °CRK | Tracks actual within 2–3°. Deviation = VVT fault. |
| IDE00048 | Exhaust cam specified position | °CRK | Same for exhaust. |
| IDE00182 | Cam adaptation intake: phase position | ° | CHAIN WEAR INDICATOR. 0° new. Warning: ±4°. Critical: ±6°. |
| IDE00184 | Cam adaptation exhaust: phase position | ° | Same function exhaust side. |
| IDE01385 | Timing chain: wear index | — | Small decimal near 0 when new. Increases with wear. |
| IDE00900 | VVT solenoid intake: duty cycle | % | 0–100% as ECU adjusts timing. |
| IDE00901 | VVT solenoid exhaust: duty cycle | % | 0–100%. |
| IDE00902 | Cam phaser intake: response time | ms | Below 500ms normal. Slow = oil pressure or phaser issue. |
| IDE00903 | Cam phaser exhaust: response time | ms | Same reference. |

## Boost & Turbo — Complete

| IDE Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| IDE00016 | Boost pressure: actual | bar | Idle vacuum: -0.2 to 0. WOT: 1.6–2.0. |
| IDE00017 | Boost pressure: specified | bar | ECU target. Tracks actual within 0.2 bar. |
| IDE00018 | Manifold pressure (post-throttle) | bar | Absolute manifold pressure. Near ambient at idle. |
| IDE00019 | Boost pressure upstream throttle | bar | Charge air pressure before throttle body. |
| IDE00396 | Wastegate actuator: acknowledgment voltage | V | EWG position feedback. 0–5V range. |
| IDE00397 | Wastegate actuator: commanded voltage | V | ECU command to EWG motor. |
| IDE00398 | Boost control: deviation | bar | Actual minus target. Near 0 ideal. Large = boost leak or EWG fault. |
| IDE00700 | Turbo speed (if equipped) | RPM | 50k–200k RPM at speed. |
| IDE00701 | Intercooler inlet temperature | °C | Ambient + 80–120°C during boost. |
| IDE00702 | Intercooler outlet temperature | °C | Near ambient when moving. |
| IDE00703 | Intercooler efficiency | % | Above 70% adequate. Below 60% = compromised. |
| IDE00704 | Diverter valve: position | — | Open / Closed / Modulating. |
| IDE00705 | Wastegate duty cycle: commanded | % | 0% = closed (max boost). 100% = open (no boost). |
| IDE00706 | Compressor inlet pressure | bar | Near ambient. Below = air filter restriction. |

## EVAP & PCV — Complete Live Data

| IDE Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| IDE00420 | EVAP purge valve N80: activation | % | 0% closed. Opens during cruise. 0–100% duty. |
| IDE00421 | EVAP purge: flow rate | g/s | Increases when purge valve open. |
| IDE00422 | Fuel tank pressure | mbar | Near 0 at ambient. Large swings during temp change. |
| IDE00423 | Canister vent valve N115: status | — | Open normally. Closed during leak test. |
| IDE00424 | EVAP system: leak test result | — | Pass / Fail / Not tested. |
| IDE00425 | EVAP purge quantity accumulated | mg | Running total vapor burned via purge. |
| IDE00480 | Crankcase pressure | mbar | -5 to -15 mbar normal. Positive = PCV blocked. |
| IDE00481 | PCV valve: status | — | Open / Closed / Modulating. |

## Oil, Electrical & Misc Engine Channels — Complete

| IDE Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| IDE00600 | Oil pressure: actual | bar | Warm idle: 1.5–3 bar. Below 1 = STOP ENGINE. |
| IDE00601 | Oil pressure sensor: voltage | V | 0.5–4.5V normal range. |
| IDE00602 | Oil level: sensor value | mm | If electronic oil level equipped. Supplement with dipstick. |
| IDE00603 | Oil temperature | °C | Warm: 100–115°C. Hot load: up to 130°C. |
| IDE00610 | Engine hours: total | H | Lifetime engine running hours. |
| IDE00611 | Idle hours: total | H | Hours spent at idle. |
| IDE00375 | Alternator load signal | % | 20–45% idle. 70–100% heavy electrical load. |
| IDE00252 | Brake vacuum pump: status | — | Cycles every 30–60 seconds at idle. Normal. |
| IDE00380 | Cooling fan 1: activation | % | 0% cold. Increases with temp. |
| IDE00381 | Cooling fan 2: activation | % | 0% normally. Active at high load or temp. |
| IDE01266 | Coolant shut-off valve | — | Open / Closed. Controls heater matrix flow. |
| IDE00322 | Basic setting status | — | Not active normally. Active during Basic Settings routine. |
| IDE00323 | Security access: invalid key count | — | 0 = no failed attempts. |
| IDE01076-MAS00109 | Time since ignition off: hours | H | Useful for hot start diagnosis. |
| IDE01076-MAS00110 | Time since ignition off: minutes | min | Precise soak time. |

# MODULE 02 — All Basic Settings Routines (DQ381)

Security Access: 01138. Engine running unless noted.

| Routine Name | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Transmission Adaptation Reset 'Adaptation reset' | Engine running. Selector in Park. No overheating. | Clears all learned shift values. Takes effect immediately. Shifts may feel unusual for 200–500 miles. Always follow with Drive-In procedure. |
| Drive-In Adaptation Procedure 'Drive in calibration' | After adaptation reset. ATF at operating temp. | Drive gently: 5x stop-to-30 mph, 3x stop-to-50 mph, 3 minutes at 40 mph, 3 minutes at 60 mph, one kick-down. Full adaptation over 200–500 miles. |
| Clutch K1/K2 Touch Point Calibration 'Clutch calibration' | New clutch installed. ATF at 60°C minimum. Flat surface. | ECU slowly engages each clutch to find mechanical engagement point. 60–120 seconds. Required after clutch replacement or mechatronic replacement. |
| Fluid Service Reset 'Fluid service reset' | After every ATF and filter change. | Resets ATF service counter. Takes 2–5 seconds. Confirm counter reset in Adaptation channels after. |
| Throttle Coordination 'Throttle coordination' | After TCM replacement. Engine running. | Slowly press throttle full travel and back. TCM re-learns throttle-to-gear-demand relationship. |
| Selector Lever Adaptation 'Selector lever' | Engine running. Foot on brake. | Cycle selector through P-R-N-D-S and back as prompted. TCM stores each mechanical detent position. |
| Transmission Oil Temperature Check 'Oil temperature check' | ATF at 35–45°C verified by live data. | Run with overflow plug removed. Confirms fill level procedure. ATF should drip slowly at correct level. |
| Launch Control Activation 'Launch control' | Sport or S-Tronic specific variant only. Engine warm. | Configures launch control engagement RPM if launch control is coded active. Not on all CZPB variants. |
| Gear Recognition Reset 'Gear recognition' | After selector or mechatronic replacement. | TCM re-identifies which gear ratios correspond to which selector/solenoid combinations. |
| Mechatronic Initialization 'Mechatronic init' | After mechatronic unit replacement. All above routines must follow. | Initial handshake between new mechatronic and TCM. Run before all other routines on new mechatronic. |

# MODULE 02 — All Adaptation Channels (DQ381)

Security Access: 01138.

| Channel Name | Read Value | Write Options | When to Modify |
| --- | --- | --- | --- |
| Single gear display D-mode 'single gear display' | Off (factory) | On / Off | Set to On: cluster shows D1–D7 while in Drive mode. Popular and useful modification. |
| Auto-hold function 'auto hold' | Active | Active / Inactive | Disables brake auto-hold when stopped. |
| Sport mode default 'sport mode default' | Last used | Comfort / Normal / Sport | Sets transmission mode at every startup. |
| Paddle shift response 'paddle response' | Standard | Fast / Standard / Slow | Adjusts how quickly transmission responds to paddle input. |
| Manual mode auto-upshift at redline 'auto upshift' | Active | Active / Inactive | Disable for full manual control at track. |
| Manual mode auto-downshift 'auto downshift' | Active | Active / Inactive | Disable for full manual. Risk of lugging engine. |
| Reverse without brake 'reverse brake' | Inactive | Active / Inactive | Allow R selection without brake pedal. Factory inactive for safety. |
| Launch control RPM 'launch RPM' | Varies | RPM value | Sets launch control hold RPM. Only if LC is coded active. |
| ATF service counter reset 'ATF counter' | Current miles/km | Set to 0 | Reset to 0 after every ATF service. |
| Clutch K1 adaptation: idle touch point 'K1 touch' | Learned value | Read-only / relearned via Basic Settings | Do not modify manually. Use Basic Settings clutch calibration. |
| Clutch K2 adaptation: idle touch point 'K2 touch' | Learned value | Read-only | Same as K1. |
| Shift program: economy bias 'economy shift' | Standard | Economy / Standard / Sport | Affects shift points in D mode. Economy = early upshifts. Sport = later upshifts. |

# MODULE 02 — All Live Data Channels (DQ381)

| Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| Current gear | Engaged gear | 1–7, R, N, P | Shows actual gear engaged. |
| Target gear | Gear TCM is selecting | 1–7, R, N | Matches current during steady state. Different during shift. |
| Selector position | Physical selector lever position | — | P / R / N / D / S |
| Drive program | Active driving mode | — | Comfort / Normal / Sport / Manual |
| Input shaft 1 speed | Odd-gear shaft speed | RPM | Matches engine RPM when K1 engaged. |
| Input shaft 2 speed | Even-gear shaft speed | RPM | Matches engine RPM when K2 engaged. |
| Output shaft speed | Transmission output | RPM | Correlates to vehicle speed. |
| Vehicle speed (TCM) | Speed as seen by TCM | km/h | Matches cluster within 1–2 mph. |
| K1 slip | Odd-gear clutch slip | RPM | 0 in steady state. >50 RPM persistent = worn clutch or fluid. |
| K2 slip | Even-gear clutch slip | RPM | Same reference. |
| K1 temperature | Odd-gear clutch pack temp | °C | 60–90°C normal. Above 130°C = cooling needed. |
| K2 temperature | Even-gear clutch pack temp | °C | Typically runs warmer than K1 in city use. |
| K1 torque transmitted | Torque through odd clutch | Nm | Varies with load and gear. |
| K2 torque transmitted | Torque through even clutch | Nm | Same. |
| ATF temperature | Transmission fluid temperature | °C | 60–90°C normal. Above 110°C = investigate. |
| System hydraulic pressure | Main hydraulic line pressure | bar | 4–6 bar idle. 8–15 bar under load. |
| Clutch K1 pressure | Hydraulic pressure K1 | bar | Varies during engagement. Erratic = solenoid fault. |
| Clutch K2 pressure | Hydraulic pressure K2 | bar | Same reference. |
| Engine torque request | Torque TCM requesting | Nm | Drops during upshifts for smooth power interruption. |
| Torque limit active | Whether TCM limiting input | — | Active / Inactive. |
| Mechatronic temperature | Internal mechatronic temp | °C | Normal: 40–80°C. High = thermal protection risk. |
| Total shifts completed | Lifetime shift count | Count | Useful for evaluating transmission wear history. |
| Adaptive shift count | Shifts since last adaptation reset | Count | Higher count = more learning has occurred. |

# MODULE 03 — All Basic Settings Routines

Security Access: 40168.

| Routine Name | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| VCDS-Assisted Brake Bleed 'Brake bleed' | Fresh fluid. Bleed kit connected. All four wheels accessible. | Cycles ABS solenoids per VCDS prompts. Sequence: RR → LR → RF → LF. Final system purge cycle. Firm pedal result. |
| Steering Angle Sensor Calibration 'SAS calibration' | Flat level surface. Wheels straight ahead. Engine running. | Stores current steering position as 0°. Verify straight-ahead tracking afterward. |
| G-Sensor / Yaw Rate Calibration 'G-sensor calibration' | Perfectly level surface. Vehicle completely stationary 60 seconds. | Stores current orientation as reference. Do not move during calibration. |
| ABS Pump Self-Test 'ABS pump test' | Vehicle stationary. Ignition ON. | Activates pump and solenoids. Listen for cycling sound. Pass = all solenoids and pump functional. |
| EPB Service Mode Open 'EPB service open' | Ignition ON. Vehicle stationary. EPB disengaged. | Retracts rear caliper pistons for pad replacement. Both rear calipers retract simultaneously. Takes 15–20 seconds. |
| EPB Service Mode Close 'EPB service close' | New pads installed. Pistons pressed against pads manually. | Returns calipers to operating position and calibrates to new pad thickness. Required after every rear pad service. |
| Auto-Hold Calibration 'Auto hold calibration' | Engine running. Level surface. Foot on brake. | Calibrates brake pressure required for automatic hold. Required after MK100 IPB replacement. |
| Pre-Sense Brake Test 'Pre-sense test' | Vehicle stationary. Engine running. | Tests Pre-sense brake preparation circuit. Verifies MK100 can apply pressure ahead of actual pedal press. |
| Brake Pad Reset (after replacement) 'Brake pad reset' | After fitting new pads. | Resets brake pad wear counter in module. Clears pad wear warning. |
| Tire Circumference Adaptation 'Tire calibration' | After changing to different tire size. | Updates ABS/ESP calculations for new tire rolling circumference. Critical for accurate ABS modulation. |
| Corner Brake Control Test 'CBC test' | Vehicle stationary. | Verifies CBC (corner brake control) pressure modulation valves function correctly. |
| Roll Stability Calibration 'Roll stability' | Vehicle stationary. Level surface. | Calibrates roll stability reference. Run after suspension component replacement. |

# MODULE 03 — All Adaptation Channels

Security Access: 40168.

| Channel Name | Default | Options | When to Modify |
| --- | --- | --- | --- |
| ESP system installed 'ESP installed' | Active | Active / Inactive | ONLY disable for track days. Re-enable immediately after. May be illegal on public roads. |
| ASR traction control 'ASR installed' | Active | Active / Inactive | Disable for track (wet/snow). Re-enable after. |
| EDL electronic diff lock 'EDL installed' | Active | Active / Inactive | Not recommended to disable. Affects low-speed traction. |
| ABS installed 'ABS installed' | Active | Active / Inactive | NEVER disable. Illegal in most jurisdictions. |
| Pre-sense brake installed 'pre-sense' | Active (if equipped) | Active / Inactive | Disable only for specific testing. Re-enable immediately. |
| Auto-hold installed 'auto hold' | Active | Active / Inactive | Disable if auto-hold hardware removed or malfunctioning. |
| Hill hold control 'HHC installed' | Active | Active / Inactive | Disable if unwanted on flat terrain primarily. |
| MSR engine drag control 'MSR installed' | Active | Active / Inactive | Controls engine braking via throttle during aggressive decel. Disable only for track. |
| ESP button hold time 'ESP disable hold' | 3 seconds | 1 / 2 / 3 / 5 sec | Shorten for easier track day ESP disabling. |
| ABS brake assist sensitivity 'brake assist' | Standard | Standard / High | High = brake assist activates more readily. |
| Hill hold: release delay 'HHC delay' | 2 seconds | 1 / 2 / 3 sec | Adjust hill hold brake release timing. |
| Tire pressure plausibility 'tire pressure check' | Active | Active / Inactive | Wheel speed comparison for deflated tire detection. |
| Emergency brake signal 'emergency brake flash' | Active | Active / Inactive | Hazard flash during emergency braking. Leave Active. |
| Tire circumference: front 'tire front' | Factory spec | Calculated value | Update after tire size change. Affects all ABS/ESP calculations. |
| Tire circumference: rear 'tire rear' | Factory spec | Calculated value | Same as front. |
| Trailer stability 'trailer stability' | Active | Active / Inactive | Active when trailer detected. Leave Active. |
| SAS calibration stored value 'steering angle offset' | 0° (calibrated) | Read-only after calibration | Read to verify calibration. Should be 0° ± 0.5° after correct calibration procedure. |

# MODULE 03 — All Live Data Channels

| Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| Wheel speed FL | Front left wheel speed | km/h | All four within 2 mph on straight road. |
| Wheel speed FR | Front right wheel speed | km/h | Outer wheel faster in turns — normal. |
| Wheel speed RL | Rear left wheel speed | km/h | Rear slightly slower than front in turns. |
| Wheel speed RR | Rear right wheel speed | km/h | Same. |
| Brake pressure sensor 1 | Master cylinder pressure | bar | 0–2 bar rest. 80–150 bar hard braking. |
| Brake pressure sensor 2 | Secondary circuit pressure | bar | Should match sensor 1. |
| Brake pedal position/switch | Pedal status | — or mm | Pressed / Released or continuous travel. |
| IPB booster pressure | MK100 reserve pressure | bar | ~175 bar when ignition ON. |
| Lateral acceleration | Side G-force | m/s² or g | 0 straight. Increases in corners. |
| Longitudinal acceleration | Forward/backward G | m/s² | 0 constant speed. Positive accel, negative braking. |
| Yaw rate | Rotation rate | °/s | 0 straight. Positive left turn. |
| Steering angle | Wheel position | ° | 0° straight. ±540° lock-to-lock. |
| ESP active | Stability intervention | — | Active / Inactive. |
| ASR active | Traction control intervention | — | Active / Inactive. |
| ABS active | ABS modulation | — | Active / Inactive. |
| EDL active | Electronic diff lock | — | Active / Inactive. |
| CBC active | Corner brake control | — | Active / Inactive. |
| MSR active | Engine drag control | — | Active / Inactive. |
| HHC active | Hill hold active | — | Active / Inactive. |
| Auto-hold active | Auto brake hold | — | Active / Inactive. |
| EPB left: position | Left rear caliper state | — | Applied / Released. |
| EPB right: position | Right rear caliper | — | Applied / Released. |
| Brake pad wear FL | Front left pad sensor | — | OK / Worn (if sensor equipped). |
| Brake pad wear FR | Front right pad sensor | — | OK / Worn. |
| Brake pad wear RL | Rear left pad sensor | — | OK / Worn. |
| Brake pad wear RR | Rear right pad sensor | — | OK / Worn. |
| Pre-sense: status | Pre-sense system state | — | Ready / Active / Fault. |
| Vehicle deceleration: actual | Current deceleration rate | m/s² | 0 constant. Negative = braking. ABS threshold ~8–10 m/s². |

# MODULE 08 — All Basic Settings Routines (Comfort System)

Security Access: 31347.

| Routine Name | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Window initialization FL 'window initialization front left' | Door fully closed. Ignition ON. | Hold DOWN until fully open. Release 3 sec. Hold UP until fully closed. Hold 3 more sec. One-touch and anti-pinch restored. |
| Window initialization FR 'window initialization front right' | Same as FL. | Same procedure as driver window. |
| Window initialization RL 'window initialization rear left' | Sportback only. | Same procedure rear left window. |
| Window initialization RR 'window initialization rear right' | Sportback only. | Same procedure rear right window. |
| Sunroof initialization 'sunroof initialization' | Ignition ON. | Hold CLOSE until fully closed. Release 3 sec. Hold OPEN to fully open. Hold CLOSE again and hold 3 extra sec. One-touch restored. |
| Rain sensor calibration 'rain sensor calibration' | Clean dry windshield at sensor location. | VCDS sets baseline optical reference for zero-rain condition. Run after windshield replacement. |
| Central locking test 'locking test' | All doors closed. | Cycles all door locks simultaneously. Verifies all actuators respond to central lock signal. |
| Mirror fold initialization 'mirror initialization' | Mirrors at neutral position. | Mirrors fold and unfold once. Stores end-stop positions for motor protection. |
| Interior lighting test 'interior lighting test' | Ignition ON. | Cycles all interior light elements. Identifies failed bulbs or LED elements. |
| Alarm system test 'alarm test' | Car locked and alarm armed. | Tests horn, hazard flash, and siren outputs without triggering full alarm protocol. |
| Wiper park position reset 'wiper park' | Wipers in motion. | Commands wipers to park position and stores it. Run if wipers stop at wrong position. |

# MODULE 08 — All Adaptation Channels

Security Access: 31347. This module has the most customizable adaptation channels on the A3.

| Channel Name | Default | Options | Effect |
| --- | --- | --- | --- |
| Comfort closing: windows 'comfort closing windows' | Inactive | Active / Inactive | Hold lock button 2+ sec to close all windows remotely. |
| Comfort opening: windows 'comfort opening windows' | Inactive | Active / Inactive | Hold unlock button 2+ sec to open all windows remotely. |
| Comfort closing: sunroof 'comfort closing sunroof' | Inactive | Active / Inactive | Sunroof closes with comfort close button. |
| Comfort opening: sunroof 'comfort opening sunroof' | Inactive | Active / Inactive | Sunroof opens with comfort open button. |
| Window close on rain 'rain close windows' | Inactive | Active / Inactive | Windows close automatically when rain sensor detects precipitation. |
| Rain close: sensitivity threshold 'rain close threshold' | Medium | Low / Medium / High | At what rain level auto-close activates. |
| Anti-pinch: force threshold 'pinch force' | Standard | Standard / High | Increase if tint causes false anti-pinch trigger. |
| Comfort open/close: key hold duration 'key hold duration' | 2 seconds | 1.5 / 2 / 2.5 / 3 sec | How long to hold key fob button to trigger comfort function. |
| Window auto-close speed 'speed close windows' | Inactive | Inactive / 80–130 km/h | Speed above which windows auto-close. Useful for highway driving. |
| Auto-lock when driving 'auto lock speed' | Inactive | Inactive / 10 / 15 / 20 km/h | Doors lock when speed reached. |
| Auto-unlock on ignition off 'auto unlock' | Active | Active / Inactive | Doors unlock automatically when ignition off. |
| Auto-unlock on crash 'crash unlock' | Active | Active / Inactive | SAFETY — leave Active. Doors unlock on airbag deploy. |
| Selective unlock: driver door first 'selective unlock' | Inactive | Active / Inactive | First key press unlocks driver only. Second unlocks all. |
| Visual confirmation: hazards flash 'visual confirmation' | Active | Active / Inactive | Hazard lights flash on lock/unlock. |
| Acoustic confirmation: horn 'acoustic horn' | Inactive | Active / Inactive | Horn beeps on lock. Loud — intrusive in quiet areas. |
| Acoustic confirmation: interior 'acoustic interior' | Inactive | Active / Inactive | Interior door sounder only. Subtle. Recommended alternative to horn. |
| Panic alarm delay 'panic delay' | Inactive | Inactive / 2 seconds | 2 sec before panic alarm sounds. Prevents accidental activation. |
| Central locking: all simultaneous 'lock simultaneous' | Active | Active / Inactive | All doors lock at same time vs sequential. |
| Soft close: installed 'soft close' | Inactive | Active / Inactive | Enable only with Audi soft-close hardware installed. |
| Mirror fold on lock 'mirror fold lock' | Inactive | Active / Inactive | Mirrors fold automatically when car locked. |
| Mirror fold speed threshold 'mirror fold speed' | 0 km/h | 0 / 10 / 15 / 20 km/h | Speed below which mirrors fold on lock. |
| Mirror fold re-extend speed 'mirror extend speed' | 15 km/h | 10 / 15 / 20 km/h | Speed at which folded mirrors auto-extend when driving. |
| Passenger mirror dip in reverse 'mirror dip reverse' | Inactive | Active / Inactive | Right mirror tilts toward curb when R selected. |
| Mirror heating: linked to defroster 'mirror heat' | Active | Active / Inactive | Mirror heaters activate with rear defroster button. |
| Rear wiper in reverse 'rear wiper reverse' | Inactive | Active / Inactive | Rear wiper activates in R when front wipers running. |
| Rain sensor sensitivity base 'rain sensitivity' | Medium | 1 (low) – 5 (high) | Baseline rain sensor responsiveness. |
| Wiper wipes after wash 'wash wipes' | 3 wipes | 1 / 2 / 3 / 4 | Number of wiper passes after releasing washer button. |
| Delayed wipe after wash 'delayed wipe' | Active | Active / Inactive | One additional wipe seconds after wash cycle. |
| Speed-dependent wiper interval 'speed wiper' | Active | Active / Inactive | Wiper interval speeds up with vehicle speed. |
| Rain sensor in car wash 'car wash mode' | Inactive | Active / Inactive | Prevents wipers activating during car wash spray. |
| Interior welcome lighting duration 'welcome duration' | 30 seconds | 10 / 20 / 30 / 60 sec | How long interior lights stay on after unlocking. |
| Interior goodbye lighting duration 'goodbye duration' | 30 seconds | 10 / 20 / 30 / 60 sec | How long lights stay on after locking. |
| Interior lighting dimming curve 'dimming curve' | Linear | Linear / Logarithmic | Logarithmic feels more natural. |
| Comfort flashing: number of flashes 'comfort flash count' | 3 | 1 / 2 / 3 / 4 / 5 | One-tap turn signal flash count. 5 recommended for highway lane changes. |
| Emergency brake flash 'emergency flash' | Active | Active / Inactive | Hazards flash during emergency braking. Leave Active. |
| Alarm: installed 'alarm installed' | Active | Active / Inactive | Master alarm enable. |
| Interior protection sensor 'interior sensor' | Active | Active / Inactive | Disable if dog in car, windows open, or false triggers. |
| Tilt sensor 'tilt sensor' | Active | Active / Inactive | Disable when on car carrier. Re-enable after transport. |
| Alarm: horn output 'alarm horn' | Active | Active / Inactive | Horn sounds during alarm. Disable for silent alarm (still flashes). |
| Alarm: siren duration 'alarm duration' | 30 seconds | 15 / 30 / 45 / 60 sec | How long alarm sounds before auto-silencing. |
| Alarm: trigger count 'alarm count' | 3 | 1 / 2 / 3 / 5 | How many alarm cycles before going silent. |
| Alarm: power loss detection 'power loss alarm' | Active | Active / Inactive | Triggers alarm if battery disconnected while armed. DISABLE before any electrical work. |
| Alarm: arming delay 'alarm delay' | 30 seconds | 0 / 15 / 30 / 60 sec | Delay between locking and full alarm arming. Allows interior sensor to initialize. |
| Heated steering wheel: installed 'steering heat' | Varies | Active / Inactive | Enable if hardware present. Must also be coded. |
| Seat heating: startup level 'seat heat startup' | Last setting | Off / Low / Last | Seat heat level at startup. |

# MODULE 08 — All Live Data Channels

| Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| Window FL: position | Driver window height | % or mm | 0% fully open, 100% fully closed. |
| Window FR: position | Passenger window | % or mm | Same. |
| Window RL: position | Rear left (Sportback) | % or mm | Same. |
| Window RR: position | Rear right (Sportback) | % or mm | Same. |
| Sunroof position | Sunroof opening | % or mm | 0% closed, various for tilt/open. |
| Door FL: status | Driver door open/closed | — | Open / Closed. |
| Door FR: status | Passenger door | — | Open / Closed. |
| Door RL: status | Rear left door | — | Open / Closed. |
| Door RR: status | Rear right door | — | Open / Closed. |
| Trunk/boot: status | Boot lid status | — | Open / Closed. |
| Hood: status | Bonnet status | — | Open / Closed. |
| Central locking: status | Lock state all doors | — | Locked / Unlocked. |
| Driver door lock individual | Driver door lock state | — | Locked / Unlocked. |
| Passenger door lock individual | Passenger door lock state | — | Locked / Unlocked. |
| Wiper park position | Wiper home state | — | In park / Not in park. |
| Rain sensor: signal level | Optical rain detection | % or raw | 0 = dry. Increases with rain intensity. |
| Light sensor: ambient level | Ambient brightness | Lux or raw | High = daylight. Low = darkness. Drives auto headlights. |
| Rain sensor: auto wiper active | Whether auto wiper running | — | Active / Inactive. |
| Mirror fold status | Mirror position | — | Folded / Extended. |
| Mirror heating: left | Left mirror heater state | — | On / Off. |
| Mirror heating: right | Right mirror heater state | — | On / Off. |
| Alarm: armed | Alarm system state | — | Armed / Disarmed. |
| Alarm: triggered | Active alarm state | — | Active / Not active. |
| Interior sensor level | Interior motion detection | Raw value | Low = no motion. Elevated = movement detected. |
| Tilt sensor value | Vehicle tilt detection | Raw or ° | Stable when parked. Change = potential wheel theft. |
| KESSY handle: driver touch | Driver handle capacitive sensor | — | Touched / Not touched. |
| KESSY handle: passenger touch | Passenger handle sensor | — | Touched / Not touched. |
| Seat heating driver: level | Driver seat heat level | 0–3 | 0 = off. 3 = maximum. |
| Seat heating passenger: level | Passenger seat heat level | 0–3 | Same. |
| Steering wheel heating: status | Heated steering active | — | On / Off (if equipped). |

# MODULE 09 — All Basic Settings Routines (Central Electronics)

Security Access: 20103.

| Routine Name | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Headlight range adjustment 'headlight level calibration' | Headlights on. Normal ride height. Driver only in seat. | Leveling motors cycle through range. ECU stores reference position. Post-check: headlights should aim symmetrically. |
| Coming/leaving home calibration 'home light calibration' | After changing duration adaptations. | Stores new timing values to module non-volatile memory. |
| Drive Select initialization 'Drive Select init' | Engine running. After TCM or Drive Select module replacement. | Resets Drive Select mode parameters across all connected modules (ECM, TCM, steering). |
| DRL calibration 'DRL calibration' | DRL system active. | Stores brightness calibration values. Run after DRL brightness adaptation changes. |
| Ambient lighting initialization 'ambient light init' | After ambient lighting changes. | Re-initializes ambient lighting color/brightness maps. |
| Cornering light calibration 'cornering light calibration' | Headlights on. Engine running. | Verifies cornering light elements activate correctly at correct steering angle threshold. |

# MODULE 09 — All Adaptation Channels

Security Access: 20103.

| Channel Name | Default | Options | Effect |
| --- | --- | --- | --- |
| DRL activation mode 'DRL mode' | Active with ignition | Active / Inactive / Active with parking lights | Controls when DRLs illuminate. |
| DRL brightness: headlights off 'DRL brightness off' | 100% | 10–100% | DRL brightness in daytime without headlights. |
| DRL brightness: headlights on 'DRL brightness on' | 30% | 0–50% | DRL brightness when headlights also active. 0% = DRLs off with headlights. |
| DRL: which element 'DRL element' | LED strip | LED strip / Parking light / Low beam reduced | Which physical light serves as DRL. |
| DRL: disable with parking brake 'DRL parking brake' | Inactive | Active / Inactive | DRLs off when parking brake applied with engine running. |
| DRL: switch off during turn signal 'DRL turn signal off' | Active | Active / Inactive | DRL on turning side extinguishes during signal. |
| DRL: dim during turn signal 'DRL dim signal' | Inactive | Active / Inactive | DRL dims to 30% during turn signal (alternative to full off). |
| Auto headlights: activation threshold 'auto light threshold' | Medium | Dark / Medium / Light | Ambient light level to trigger auto headlights. |
| Auto headlights: tunnel detection 'tunnel detection' | Active | Active / Inactive | Immediate activation when rapid light level drop (tunnel entry). |
| Auto headlights: rain activation 'auto light rain' | Inactive | Active / Inactive | Headlights activate automatically when rain sensor detects rain. |
| Auto headlights: shutoff delay 'light shutoff delay' | 10 seconds | 0 / 5 / 10 / 20 / 30 sec | Delay after ambient light increases before headlights turn off. |
| Coming home: duration 'coming home' | 30 seconds | 0 / 10 / 20 / 30 / 60 sec | Headlight duration after engine off and door opened. 0 = disabled. |
| Leaving home: duration 'leaving home' | 30 seconds | 0 / 10 / 20 / 30 / 60 sec | Headlight duration when unlocking at night. |
| Cornering lights: installed 'cornering lights' | Varies by spec | Active / Inactive | Must match hardware. |
| Cornering lights: speed threshold 'cornering speed' | 40 km/h | 0 / 20 / 40 / 60 km/h | Maximum speed for cornering light activation. |
| Cornering lights: steering threshold 'cornering angle' | 20° | 10 / 20 / 30° | Steering input required to activate cornering light. |
| Fog lights as cornering lights 'fog cornering' | Inactive | Active / Inactive | Use front fogs as cornering lights on halogen cars. |
| Front fog lights: installed 'front fog' | Varies | Active / Inactive | Match to hardware. |
| Rear fog light: side 'rear fog side' | Passenger (North America) | Driver / Passenger | Which side the single rear fog is on. |
| Headlight washers: installed 'headlight washer' | Varies | Active / Inactive | Match to hardware. LED/xenon cars typically. |
| Headlight wash: activation interval 'wash interval' | Every 5 wipes | Every 1 / 3 / 5 washes | How often headlight washer activates with front washer. |
| Ambient lighting: installed 'ambient installed' | Varies | Active / Inactive | Match to hardware. |
| Ambient: Drive Select integration 'ambient DS' | Active | Active / Inactive | Ambient color changes with Drive Select mode. |
| Ambient: Comfort mode color 'ambient comfort' | White | White / Blue / Red / Various | Interior LED color in Comfort mode. |
| Ambient: Dynamic mode color 'ambient dynamic' | Red | White / Blue / Red / Various | Interior LED color in Dynamic mode. |
| Ambient: brightness level 'ambient brightness' | Medium | 1–7 | Overall brightness. |
| Ambient: disable in daytime 'ambient day disable' | Inactive | Active / Inactive | Only illuminates at night with headlights. |
| Drive Select: installed 'Drive Select' | Active | Active / Inactive | Master DS enable. |
| Drive Select: Efficiency mode 'efficiency mode' | Inactive (NA) | Active / Inactive | Adds Efficiency mode to DS menu. Also need Module 01 and 02 enabled. |
| Drive Select: Individual mode 'individual mode' | Active | Active / Inactive | Allow mixed settings across modes. |
| Drive Select: default startup mode 'DS default' | Last used | Comfort / Auto / Dynamic / Individual | Which mode activates at every engine start. |
| Drive Select: ADS suspension 'ADS installed' | Varies | Active / Inactive | DCC adaptive suspension integration. Match to hardware. |
| Drive Select: steering integration 'DS steering' | Active | Active / Inactive | Whether DS changes steering weight. |
| Rear lights: left brake type 'rear brake left' | Halogen | Halogen / LED | Update after LED brake light retrofit. |
| Rear lights: right brake type 'rear brake right' | Halogen | Halogen / LED | Same right side. |
| Rear lights: left turn type 'rear turn left' | Halogen | Halogen / LED | Update for LED turn signal retrofit. |
| Rear lights: right turn type 'rear turn right' | Halogen | Halogen / LED | Same right side. |
| Rear lights: license plate type 'license plate type' | Halogen | Halogen / LED | Update after LED license plate retrofit. Prevents bulb fault. |
| Reversing light: LED 'reversing LED' | Halogen | Halogen / LED | Change if retrofitting LED reversing lights. |
| LED matrix: installed 'matrix LED' | Varies | Active / Inactive | Match to hardware. Only if physical matrix LED present. |
| LED sequential turn signal 'sequential LED' | Varies | Active / Inactive | Only for cars with sequential LED hardware. |

# MODULE 09 — All Live Data Channels

| Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| Low beam: status | Headlights on/off | — | On / Off. |
| High beam: status | High beam state | — | On / Off. |
| DRL: status | DRL active state | — | On / Off. |
| DRL: brightness current | Current DRL intensity | % or raw | 100% daytime, dimmed with headlights. |
| Front fog left: status | Left fog light | — | On / Off. |
| Front fog right: status | Right fog light | — | On / Off. |
| Rear fog: status | Rear fog light | — | On / Off. |
| Turn signal left: status | Left indicator | — | On / Off / Flashing. |
| Turn signal right: status | Right indicator | — | On / Off / Flashing. |
| Hazard lights: status | Hazard state | — | Active / Inactive. |
| Reversing lights: status | Reverse lights | — | On / Off. |
| License plate lights: status | Plate illumination | — | On / Off. |
| Headlight leveling: position actual | Current aim position | % or steps | Adjusts based on load. |
| Headlight leveling: position target | Target aim | % or steps | Should track actual. |
| Ambient lighting: active | Interior ambient | — | On / Off. |
| Ambient lighting: current color | Active color | RGB or name | Shows selected color. |
| Ambient lighting: brightness | Current brightness | 0–100% | Varies with mode. |
| Auto headlights: mode active | Auto mode engaged | — | Active when switch in Auto position. |
| Light sensor: value | Ambient light level | Lux or raw | Low values trigger auto headlights. |
| Drive Select mode: current | Active DS mode | — | Comfort / Auto / Dynamic / Individual / Efficiency. |
| Drive Select: engine response | Throttle map | — | Comfort / Normal / Sport. |
| Drive Select: transmission response | Shift program | — | Comfort / Normal / Sport. |
| Drive Select: steering response | EPS weight setting | — | Soft / Medium / Heavy. |
| Drive Select: suspension mode | DCC setting (if equipped) | — | Comfort / Auto / Dynamic. |
| Cornering light left: active | Left cornering light | — | Active / Inactive. |
| Cornering light right: active | Right cornering light | — | Active / Inactive. |
| Coming home: active | Coming home sequence running | — | Active / Inactive. |
| Leaving home: active | Leaving home running | — | Active / Inactive. |
| Battery voltage at module 09 | Supply voltage | V | 12.4–14.4V. Low = charging issue. |

# MODULE 17 — All Basic Settings Routines (Instrument Cluster)

Security Access: 11046.

| Routine Name | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Oil service reset 'oil service' | After oil and filter change. | Resets oil change countdown timer and distance counter to full interval. |
| Inspection service reset 'inspection reset' | After full annual service. | Resets annual inspection countdown to factory interval. |
| TPMS calibration 'tire pressure calibration' | All tires at correct cold inflation pressure. | Stores current pressures as reference baseline. Extinguishes TPMS warning after drive. |
| Brake fluid service reset 'brake fluid reset' | After brake fluid replacement. | Resets brake fluid age counter. |
| Cluster display test 'gauge sweep' or 'display test' | Ignition ON. Engine not required. | All warning lights illuminate. All gauges sweep to max and return. 10–15 seconds. |
| Needle sweep on startup 'needle sweep' | After enabling adaptation channel. | Configures whether gauges sweep dramatically on startup (visual effect). |
| Pollen filter reset 'pollen filter' or 'cabin filter' | After replacing cabin air filter. | Resets cabin filter service counter. |
| Air filter service reset 'air filter reset' | After replacing engine air filter (if tracked). | Resets engine air filter counter if module tracks this. |
| Spark plug service reset 'spark plug reset' | After spark plug replacement (if tracked). | Resets spark plug counter if module tracks this. |
| Transmission fluid reset in cluster 'transmission fluid cluster' | After DQ381 ATF service. | Cluster-side reset of transmission service indicator. |

# MODULE 17 — All Adaptation Channels

Security Access: 11046 (most), 38041 (some).

| Channel Name | Default | Options | Effect |
| --- | --- | --- | --- |
| Speed display units 'speed units' | mph (NA) | mph / km/h | Changes speedometer primary display. |
| Distance units 'distance units' | Miles | Miles / Kilometers | Changes odometer and trip display. |
| Temperature units 'temperature units' | °F (NA) | °F / °C | Changes all temperature displays. |
| Fuel consumption units 'fuel consumption units' | mpg (NA) | mpg / L/100km / km/L | Changes fuel economy display. |
| Pressure units (TPMS) 'pressure units' | PSI | PSI / bar / kPa | Changes tire pressure display. |
| Service interval mode 'service mode' | Flexible | Fixed / Flexible | Flexible = condition-based. Fixed = set interval. |
| Oil interval: fixed distance 'oil distance' | 5,000 miles | 1,000–15,000 miles | Fixed-mode oil change distance interval. |
| Oil interval: fixed time 'oil time' | 365 days | 180–730 days | Fixed-mode time interval for oil change reminder. |
| Inspection interval: distance 'inspection distance' | 15,000 miles | 10,000–20,000 miles | Major service distance interval. |
| Inspection interval: time 'inspection time' | 730 days | 365–730 days | Major service time interval. |
| Time since last oil change 'oil time counter' | Counter in days | Set to 0 | Direct reset of time counter. More precise than Basic Settings reset. |
| Distance since last oil change 'oil distance counter' | Counter in miles/km | Set to 0 | Direct reset of distance counter. |
| Time since last brake fluid 'brake fluid counter' | Counter in days | Set to 0 | Reset brake fluid age counter. |
| Low fuel warning threshold 'low fuel' | ~2 gallons | Adjustable liters/gallons | Fuel level to trigger warning. Keep above 1.5 gallons minimum. |
| Gear shift indicator 'shift indicator' | Active | Active / Inactive | Disable upshift recommendation arrow if distracting. |
| Seatbelt warning 'seatbelt warning' | Active | Active / Inactive | Disable with caution — may be illegal. Re-enable recommended. |
| Door open warning: engine running 'door chime' | Active | Active / Inactive | Chime when door opens with ignition on. |
| Speed warning: threshold 'speed warning' | Inactive | Inactive / set speed | Alert when speed exceeds threshold. Useful for limit awareness. |
| Speed warning: second threshold 'speed warning 2' | Inactive | Inactive / set speed | Optional second speed alert at different value. |
| TPMS installed 'TPMS installed' | Varies | Active / Inactive | Must match hardware. |
| TPMS display type 'TPMS type' | Varies | Indirect / Direct | Match to installed system. |
| Service reminder display mode 'service display' | Popup on startup | Popup / Indicator only / Off | How service reminders are shown. |
| Virtual Cockpit startup view 'VC startup' | Classic | Classic / Infotainment / Sport | AVC-equipped cars only. |
| VC performance data overlay 'VC performance' | Inactive | Active / Inactive | Shows g-force, lap timer in Sport AVC mode. |
| Lane departure in cluster 'LDA display' | Active (if equipped) | Active / Inactive | Shows lane departure warning in DIS. |
| Compass display 'compass' | Active (if equipped) | Active / Inactive | Navigation compass in cluster. |
| Fuel range calculation 'range calculation' | Active | Active / Inactive | Shows remaining range estimate in DIS. |
| Average consumption reset 'avg reset' | Manual | Manual / Auto on refuel | Resets average fuel economy. Auto resets when refueled. |
| Clock format 'clock format' | 12h (NA) | 12h / 24h | Changes clock display format. |
| Oil level display in DIS 'oil level display' | Active (if sensor equipped) | Active / Inactive | Shows electronic oil level reading in DIS menu. |

# MODULE 17 — All Live Data Channels

| Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| Oil change: days remaining | Countdown to oil service | Days | Negative = overdue. |
| Oil change: distance remaining | Distance until oil service | miles/km | Negative = overdue. |
| Inspection: days remaining | Annual service countdown | Days | 2-year interval standard. |
| Inspection: distance remaining | Service distance left | miles/km | Counts down from interval. |
| Service interval mode active | Which mode is currently active | — | Fixed / Flexible. |
| Total odometer | Lifetime mileage | miles or km | Read-only. Cannot be modified. |
| Trip meter A | Current trip A | miles or km | Reset via stalk button. |
| Trip meter B | Current trip B | miles or km | Second trip counter. |
| Average fuel consumption | Economy since reset | mpg or L/100km | Running average. |
| Instantaneous fuel consumption | Real-time economy | mpg or L/100km | Changes with throttle input. |
| Average speed | Speed since reset | mph or km/h | Running average. |
| Driving time since reset | Elapsed drive time | Hours:Minutes | Since last reset. |
| Coolant temp: cluster display | As shown on gauge | °C or °F | Should match Module 01 live data. |
| Fuel level: cluster display | As shown on gauge | L or gallons | Cross-check with Module 01 fuel level. |
| Speed: cluster display | As shown on speedometer | mph or km/h | May differ slightly from ABS actual speed. |
| TPMS FL: pressure | Front left tire pressure | PSI or bar | Target: 36–38 PSI. Verify with gauge. |
| TPMS FR: pressure | Front right tire pressure | PSI or bar | Same. |
| TPMS RL: pressure | Rear left tire pressure | PSI or bar | Check door label for rear spec. |
| TPMS RR: pressure | Rear right tire pressure | PSI or bar | Same. |
| TPMS: warning active | Tire pressure warning state | — | Active / Inactive. |
| TPMS: calibration needed | Whether calibration required | — | Yes / No. |
| Check engine light: status | CEL state as seen by cluster | — | On / Off. |
| ABS warning: status | ABS light in cluster | — | On / Off. |
| Service due indicator | Service reminder lamp | — | On / Off. |
| Needle sweep: active | Whether startup sweep running | — | Active / Inactive. Active during startup animation. |
| Speed warning: triggered | Whether speed alert active | — | Active / Inactive. |
| Brake fluid age: days | Time since last brake fluid change | Days | Increases over time. Reset after service. |
| Pollen filter: service status | Cabin filter change indicator | — | OK / Due. |

— End of Exhaustive Basic Settings / Adaptations / Live Data Reference —

Every routine, every channel, every parameter for Modules 01, 02, 03, 08, 09, 17 on the 2018 Audi A3 8V MQB CZPB