VCDS COMPLETE REFERENCE — EXHAUSTIVE EDITION VOL 2

2018 Audi A3 8V MQB — EA888 Gen3 CZPB

All Basic Settings | All Adaptations | All Live Data — Modules 13 | Climate | 61 | 36 | 16 | 44 | 5F | 15 | 19 | 65 | 69 | 76

# MODULE 13 — All Basic Settings Routines (ACC / J428 Radar)

No security access required for calibration routines.

| Routine Name | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| ACC Sensor Factory Reset 'Sensor factory reset' or 'Radar factory reset' | Faults cleared. Adaptation 'sensor operational state' reset to 0 first. Engine running. | Wipes stored calibration and alignment data. Module returns to 'uncalibrated but not faulted' state. Run immediately before calibration routine. |
| ACC Calibration 'Radar calibration' or 'Distance sensor calibration' | Flat level surface. Wheels straight ahead. 10m clear ahead. Engine running. | Module reads radar return pattern. Stores alignment offset. Takes 15–60 sec. ADP OK = complete. Verify: Calibration Status = Calibrated in live data. |
| Sensor Initialization 'Sensor initialization' | After connector work or power interruption. Engine running. | Re-initializes sensor self-check registers. Clears temporary fault flags. Run before calibration if calibration rejects immediately. |
| ACC System Self-Test 'System self-test' | No faults in Module 13. Engine running. | Tests all ACC system components: radar, speed input, brake interface, engine interface. Reports pass/fail for each sub-system. |
| Front Assist Sensitivity Test 'Front assist test' | Engine running. No active Front Assist faults. | Verifies Front Assist warning thresholds are responding correctly to test targets. |

# MODULE 13 — All Adaptation Channels

| Channel Name | Default | Options | Effect |
| --- | --- | --- | --- |
| Sensor operational state 'sensor state' | Calibrated (normal) | 0 = reset / 1 = calibrated | KEY CHANNEL. Set to 0 to unlock module from misalignment fault before calibration. Must be reset before calibration will accept. |
| Calibration status 'calibration status' | Calibrated / Not calibrated | Read-mostly | Shows current calibration state. Changes to Calibrated after successful Basic Settings routine. |
| Horizontal alignment stored offset 'alignment offset horizontal' | Within ±1.5° | Read after calibration | Verify after calibration. Values outside ±3° = calibration surface issue. |
| Vertical alignment stored offset 'alignment offset vertical' | Near 0° | Read after calibration | Should be near 0° after calibration on level ground. |
| Front Assist: warning sensitivity 'front assist sensitivity' | Standard | Early / Standard / Late | Early = warning activates sooner. Late = warning activates closer to potential impact. |
| Front Assist: installed 'front assist installed' | Active (if equipped) | Active / Inactive | Must match hardware and software capability. |
| AEB: autonomous emergency braking 'AEB installed' | Active (if equipped) | Active / Inactive | Do not disable — safety system. Only disable if troubleshooting AEB false activation. |
| ACC: following distance levels 'following distance' | 5 levels | 1–5 levels available | Number of distance settings presented to driver. Reduce if fewer options preferred. |
| Traffic Jam Assist: installed 'traffic jam' | Active (if equipped) | Active / Inactive | Stop-and-go ACC. Only present if hardware supports it. |
| ACC: maximum speed 'ACC max speed' | Factory max (speed limiter) | Adjustable km/h value | Maximum speed ACC will hold. Should not exceed vehicle speed limiter. |
| ACC: resume speed after stop 'resume speed' | Last set speed | Last set / Minimum speed | Behavior when resuming from stopped: resume to last set speed or to current speed. |

# MODULE 13 — All Live Data Channels

| Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| ACC system status | Overall operational state | — | Ready / Active / Fault / Disabled. |
| Calibration status | Sensor calibration state | — | Calibrated / Not calibrated / Misaligned. |
| Calibration confidence | Calibration quality percentage | % | Above 90% = well calibrated. |
| Horizontal alignment offset | Current horizontal beam offset | ° | Within ±1.5° = calibrated. |
| Vertical alignment offset | Vertical beam offset | ° | Near 0°. |
| Misalignment fault: active | Hard misalignment lock state | — | Not active = normal. Active = must reset before calibration. |
| Basic setting active | Calibration routine running | — | Active during calibration. Not active normally. |
| Sensor operational status | Hardware status | — | Operational / Fault. |
| Target detected | Vehicle ahead tracked | — | Target present / No target. |
| Target distance | Distance to tracked vehicle | m | 0–150m. Active following starts around 100m. |
| Target relative velocity | Speed difference to target | km/h | Positive = target pulling away. Negative = approaching. |
| Target lateral position | Target offset from center | m | Near 0 = directly ahead. >1.5m = adjacent lane. |
| Target classification | Type of detected object | — | Vehicle / Unknown. |
| ACC active | Cruise system engaged | — | Active / Inactive. |
| ACC set speed | Programmed cruise speed | km/h or mph | Driver-selected speed. |
| ACC following distance setting | Selected gap level | 1–5 bars | Driver-selected distance. |
| System inhibit reason | Why ACC is currently blocked | — | Shows specific reason if ACC cannot engage. |
| Vehicle speed from ABS | Speed received from Module 03 | km/h | Must match actual speed. |
| Steering angle received | Angle from Module 44 | ° | Used for curve prediction. |
| Yaw rate received | Rate from Module 03 | °/s | Combined with steering for path prediction. |
| Brake status | Driver brake pedal state | — | Pressed / Not pressed. Pressed disengages ACC. |
| Accelerator status | Driver throttle state | — | Active / Inactive. Active overrides ACC. |
| Supply voltage to J428 | Power supply to sensor | V | 12.5–14.5V. Low = check fuse. |
| Front Assist: warning active | Collision warning state | — | Active / Inactive. |
| AEB: braking applied | Autonomous braking active | — | Active / Inactive. |
| ACC time since last calibration | Age of calibration | Hours / Days | Calibration degrades if car is involved in any front impact. |

# Climate Control — All Basic Settings Routines

No security access typically required.

| Routine Name | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Blower motor calibration 'blower calibration' | Engine running. | Blower cycles from min to max speed. Stores endpoints. Duration 30–60 sec. Run after blower replacement. |
| Air flap calibration 'flap calibration' | Engine running. | All air distribution flaps cycle full range. 30–60 sec. Clicking sounds normal. Run after manifold or actuator replacement. |
| Refrigerant level check 'refrigerant check' | Engine running. AC on. | ECU reads high/low pressure vs temperature-corrected targets. Reports OK or low. Not a substitute for manifold gauge set. |
| Pollen filter reset 'pollen filter reset' | After cabin air filter replacement. | Resets filter age counter. Takes 2–5 seconds. |
| Temperature sensor calibration 'temperature calibration' | Known accurate temperature reference available. | Allows offset correction if interior temperature sensor reads consistently high or low. |
| Compressor clutch test 'compressor test' | Engine running. AC off. | Engages and disengages compressor clutch once. Audible click confirms electrical function. |
| Seat heater test 'seat heater test' | Ignition ON. | Activates driver and passenger seat heaters at all three levels in sequence. Verifies all elements. |
| Steering wheel heater test 'steering heater test' | If equipped. | Activates steering wheel heating element. Verifies circuit. |
| Rear defroster test 'defroster test' | Ignition ON. | Activates rear window heating element. Check for warm glass within 60 seconds. |

# Climate Control — All Adaptation Channels

| Channel Name | Default | Options | Effect |
| --- | --- | --- | --- |
| AC compressor: behavior with engine 'AC behavior' | Manual | Manual / Always on / Always off | Always on = dehumidification always active. Always off = compressor never engages. |
| Dual zone: sync mode 'zone sync' | Independent | Independent / Synced | Synced: passenger mirrors driver temperature. |
| Temperature display units 'climate temperature units' | °F (NA) | °F / °C | Changes climate panel temperature display. |
| Recirculation: maximum duration 'recirculation duration' | 3 minutes | 1 / 3 / 5 / 10 min | Auto-switch back to fresh air after this duration. |
| Recirculation: auto with AC 'recirculation AC' | Active | Active / Inactive | Auto-selects recirculation when AC running hard. |
| Rear defroster: auto timeout 'defroster timeout' | 15 minutes | 10 / 15 / 20 min / Never | Auto-shutoff for rear defroster. |
| Seat heating: startup behavior 'seat heat startup' | Last setting | Off / Low / Last setting | Seat heat level on startup. |
| Climate: startup intensity 'startup intensity' | Medium | Low / Medium / High | How aggressively climate works on startup before reaching target. |
| Solar sensor: installed 'solar sensor' | Active (if equipped) | Active / Inactive | Match to hardware. |
| Solar sensor: correction factor 'solar correction' | Factory | Adjustable offset | Fine-tunes solar load input effect on auto climate. |
| Air quality sensor: installed 'air quality' | Inactive (most NA) | Active / Inactive | Enable only with hardware present. Activates auto recirculation in pollution. |
| Pollution threshold: auto recirculation 'pollution threshold' | Medium | Low / Medium / High | Pollution level triggering auto recirculation. |
| Fragrance dispenser: installed 'fragrance' | Inactive | Active / Inactive | Enable only with hardware installed. |
| Start-stop: climate integration 'climate start stop' | Active | Active / Inactive | Climate reduces blower during engine stop for battery. |
| Rear AC vents: installed 'rear vents' | Varies | Active / Inactive | Enable if Sportback rear climate vents installed. |
| Compressor: maximum speed limit 'compressor limit' | Factory | Adjustable | Limits compressor speed to protect system. Do not increase above factory. |

# Climate Control — All Live Data Channels

| Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| Evaporator temperature | AC evaporator core temp | °C | 2–8°C when AC running. Below 0°C = icing risk. |
| Refrigerant pressure: high side | High pressure circuit | bar | AC on: 12–18 bar. |
| Refrigerant pressure: low side | Low pressure circuit | bar | AC on: 1.5–3 bar. |
| Compressor clutch: engaged | Compressor state | — | Engaged / Disengaged. |
| Compressor: speed or displacement | Compressor output | % or RPM | Variable: 0–100%. Higher = more cooling demand. |
| Ambient temperature | Outside air temp | °C | Should match displayed ambient temp. |
| Interior temp: driver zone | Driver side cabin temp | °C | Should track actual cabin temperature. |
| Interior temp: passenger zone | Passenger side temp | °C | Dual-zone cars only. |
| Set temperature: driver | Driver's selected temp | °C or °F | Whatever is set on panel. |
| Set temperature: passenger | Passenger selected temp | °C or °F | Dual-zone only. |
| Blower speed: actual | Fan motor speed | RPM or level 0–8 | Auto mode: varies with demand. |
| Blower speed: requested | Target blower speed | RPM or level | Should match actual. |
| Air distribution flap: dash | Dash vent activation | % or on/off | Active when dash vents selected. |
| Air distribution flap: feet | Foot vent activation | % or on/off | Active for floor heat. |
| Air distribution flap: screen | Defrost mode activation | % or on/off | Active for windscreen defrost. |
| Fresh/recirculation mode | Current air source | — | Fresh / Recirculation. |
| Rear defroster: active | Rear heater state | — | Active / Inactive. |
| Seat heat driver: level | Driver seat heater | 0–3 | 0=off, 1–3=increasing heat. |
| Seat heat passenger: level | Passenger heater | 0–3 | Same scale. |
| Steering wheel heat: active | Heated steering state | — | Active / Inactive. |
| Solar sensor: radiation value | Solar load input | W/m² | Higher = more sun. Climate compensates. |
| Air quality sensor: value | Pollution level detected | PPM or raw | Higher = more pollution = recirculation activates. |
| Pollen filter: service indicator | Filter age status | — | OK / Service due. |
| Temperature flap: driver position | Driver blend flap position | % or ° | 0% = full cold. 100% = full hot. |
| Temperature flap: passenger position | Passenger blend flap position | % or ° | Same reference. |
| Fresh air flap: position | Recirculation/fresh flap | % or position | 0% = recirculation. 100% = full fresh. |

# MODULE 61 — All Basic Settings Routines (Battery Management)

| Routine Name | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Battery registration 'battery registration' | New battery installed. Battery specs known (Ah, CCA, type, manufacturer). | Consolidated routine to set all battery parameters in one step. Preferred over individual adaptation channels. Resets all counters. |
| Start-stop adaptation reset 'start-stop reset' | After battery registration. | Clears start-stop learned conditions. Allows start-stop to recalibrate to new battery over 2–3 drive cycles. |
| Alternator output test 'alternator test' | Engine running. | Commands alternator to several output voltages and verifies battery response. Tests voltage regulator function. |
| Battery capacity test 'battery capacity test' | Engine off. Battery at rest. | Applies controlled load and measures voltage response. Estimates actual remaining capacity vs rated capacity. More accurate than SoH reading alone. |
| Parasitic draw test mode 'quiescent current test' | Vehicle parked. All doors closed. Engine off for 30 min. | Puts vehicle in diagnostic sleep mode. Measures total quiescent current draw via battery sensor. Normal: below 50 mA after sleep. |

# MODULE 61 — All Adaptation Channels

| Channel Name | Default/Current Value | Write Options | When to Modify |
| --- | --- | --- | --- |
| Battery type 'battery type' | AGM (most CZPB) | AGM / EFB / Standard flooded | Match to installed battery. AGM requires different charge strategy. |
| Battery capacity 'battery capacity' | 70 or 80 Ah | 40–110 Ah | Enter exact Ah from label after every replacement. |
| Battery manufacturer 'battery manufacturer' | OEM / VW | Various brands dropdown | Select actual brand. Allows manufacturer-specific charge optimization. |
| Battery CCA rating 'battery CCA' | 680 or 800 A | Enter value | Enter from battery label. Affects start-stop minimum reserve calculations. |
| Battery replacement flag 'battery replace' | No | Yes / No | Set to Yes during registration to reset all counters. Returns to No automatically after reset completes. |
| Battery total charge throughput 'charge throughput' | 0 after registration | Read / reset via registration | Cumulative Ah charged into battery. Should be near 0 after fresh registration. High value on new battery = BMS not reset. |
| Number of engine starts since registration 'start count' | 0 after registration | Read-only | Running count. Useful to verify registration was performed recently. |
| Start-stop: master enable 'start-stop enable' | Active | Active / Inactive | Permanently disable start-stop function. |
| Battery temperature correction 'battery temp correction' | Factory | Adjustable offset | Fine-tunes charging voltage compensation for temperature. Only adjust if battery consistently under/over charges. |
| Start-stop: minimum SoC threshold 'SS minimum SoC' | 75% | 60–80% | Minimum battery charge % before start-stop is allowed. Lower = start-stop more permissive. |
| Start-stop: minimum temperature 'SS min temp' | -5°C | Adjustable | Battery temperature below which start-stop is disabled. Raise if start-stop unwanted in cold weather. |
| Charging voltage: minimum 'min charge voltage' | Factory default | Adjustable | Minimum alternator output voltage. Do not modify unless specific calibration need. |
| Charging voltage: maximum 'max charge voltage' | Factory default | Adjustable | Maximum alternator output voltage. Do not exceed 14.8V. |

# MODULE 61 — All Live Data Channels

| Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| Battery voltage | Terminal voltage | V | Engine off: 12.4–12.8V (AGM full). Running: 13.6–14.8V. |
| Battery current | Current flow | A | Positive = charging. Negative = discharging. Start: -100 to -300A briefly. |
| Battery temperature | Battery temp at sensor | °C | Ambient at battery location. Used for charge correction. |
| Battery SoC | State of charge | % | 100% = full. Below 75% = start-stop may disable. |
| Battery SoH | State of health vs new | % | New: 100%. Below 70% = consider replacement. |
| Battery SoF | State of function (start capability) | % | Ability to crank engine. Below 60% = weak cranking. |
| Alternator: commanded voltage | Target charging voltage | V | ECU commands 12.8–14.8V in managed charging cycles. |
| Alternator: actual voltage | Measured alternator output | V | Should match commanded within 0.2V. |
| Alternator: load signal | % | Alternator output utilization | 0% = no demand. 100% = full output. |
| Start-stop: enabled | SS operational state | — | Enabled / Disabled. |
| Start-stop: disable reason | Why SS is currently off | — | Low SoC / Too cold / AC demand / Not in Park / Door open / etc. |
| Start-stop: number of cycles | How many times engine has auto-stopped | Count | Running total. High number = SS actively used. |
| Battery total charge throughput | Cumulative Ah into battery | Ah | Increases over lifetime. Resets on registration. |
| Number of starts since registration | Engine starts post-registration | Count | Increases with each start. Resets on registration. |
| Charging current: actual | Current alternator is providing | A | Positive always when running. Higher when battery needs charging. |
| Quiescent current draw | Parasitic draw during sleep | mA | Normal: 30–50 mA after 30 min sleep. Above 100 mA = parasitic draw issue. |
| Battery age: days | Days since battery registration | Days | Increases from registration date. Use to track battery age. |
| Energy flow: direction | Net energy into or out of battery | — | Charging / Discharging / Balanced. |
| Start-stop: temperature threshold met | Temperature condition for SS | — | Met / Not met. Not met when too cold. |
| Generator: enabled by BMS | Whether BMS is commanding alternator | — | Active / Inactive. BMS manages charge in pulses. |

# MODULE 36 — All Basic Settings & Adaptations (Electric Seat)

| Routine Name | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Seat initialization 'seat init' | Clear space for seat to move fully. | Moves seat to full rear limit then full forward limit. Teaches motor end-stops. Restores correct position memory. |
| Lumbar initialization 'lumbar init' | Lumbar equipped cars. | Cycles lumbar from full deflated to fully inflated. Stores pump end-points. |
| Easy entry calibration 'easy entry calibration' | After enabling easy entry adaptation. | Defines the exact rearward position for easy entry. VCDS sets this based on current seat position when routine is run. |

# MODULE 36 — All Adaptation Channels

| Channel Name | Default | Options | Effect |
| --- | --- | --- | --- |
| Easy entry: installed 'easy entry' | Inactive | Active / Inactive | Seat moves back on engine off/door open. |
| Easy entry: distance 'easy entry distance' | 80 mm | 20–150 mm | How far seat slides back for entry. |
| Easy entry: trigger 'easy entry trigger' | Ignition off + door open | Ign off only / Door only / Both | What event triggers easy entry movement. |
| Easy entry: return condition 'easy entry return' | Ignition on | Ignition on / Brake pressed / Both | What triggers seat return to driving position. |
| Memory recall: automatic on unlock 'auto recall' | Inactive | Active / Inactive | Memory position auto-recalls when key fob unlocks car. |
| Memory: key fob 1 position 'key 1 memory' | Not linked | Not linked / Position 1 / 2 / 3 | Which seat memory auto-recalls for key fob 1. |
| Memory: key fob 2 position 'key 2 memory' | Not linked | Not linked / Position 1 / 2 / 3 | Which position for key fob 2. Different drivers = different positions. |
| Mirror memory: linked to seat 'mirror memory' | Active (if memory mirrors) | Active / Inactive | Mirror angles stored/recalled with seat memory positions. |
| Seat: maximum forward limit 'forward limit' | Factory | Adjustable mm | Prevent seat from going fully forward (safety distance from steering wheel). Do not reduce below safe minimum. |
| Lumbar support: maximum inflation 'lumbar max' | Factory | Adjustable | Maximum lumbar pump pressure. Do not exceed factory value. |
| Seat position on D selection 'D position recall' | Inactive | Active / Inactive | Seat returns to driving position when D is selected (complements easy entry). |

# MODULE 36 — All Live Data Channels

| Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| Seat: fore/aft position | Forward-backward track position | mm | 0 = full rear. Maximum = full forward. |
| Seat: height front | Front cushion height | mm or steps | Current vertical front edge position. |
| Seat: height rear | Rear cushion height | mm or steps | Current vertical rear edge position. |
| Seat: backrest angle | Recline angle | ° or steps | Current backrest position. |
| Lumbar: inflation level | Lumbar support state | Steps 0–5 | 0=flat. 5=maximum support. |
| Memory 1: position stored | Whether M1 has saved data | — | Stored / Empty. |
| Memory 2: position stored | Whether M2 has saved data | — | Stored / Empty. |
| Memory 3: position stored | Whether M3 has saved data | — | Stored / Empty. |
| Motor fore/aft: status | Fore/aft motor activity | — | Moving / Stopped. |
| Motor height front: status | Front height motor | — | Moving / Stopped. |
| Motor height rear: status | Rear height motor | — | Moving / Stopped. |
| Motor backrest: status | Backrest motor | — | Moving / Stopped. |
| Easy entry: active | Easy entry sequence running | — | Active / Inactive. |
| Seat occupancy | Whether seat is occupied | — | Occupied / Unoccupied. Used with airbag system. |
| Seat heater: current level (if Module 36 controlled) | Heat level | 0–3 | Some variants control heat from Module 36 instead of climate. |

# MODULE 16 — All Basic Settings & Adaptations (Steering Column / KESSY)

| Routine Name | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Key adaptation 'key adaptation' | Existing key in car. New unprogrammed key present. Security Access 17956. | Programs new transponder to immobilizer. Required for every new key fob. Takes 30 sec per key. |
| KESSY antenna test 'KESSY test' | Engine off. Key fob inside car. | Tests all KESSY antenna zones (driver handle, passenger handle, boot handle, interior). Verifies detection range. |
| Steering column lock test 'column lock test' | Ignition off. | Cycles the electronic column lock. Verifies lock engages and releases correctly. |
| Wiper stalk calibration 'wiper calibration' | After stalk replacement. | Recalibrates wiper stalk resistance values for each position. |
| Turn signal stalk calibration 'signal calibration' | After stalk replacement. | Recalibrates turn signal stalk positions. |

# MODULE 16 — All Adaptation Channels

| Channel Name | Default | Options | Effect |
| --- | --- | --- | --- |
| KESSY: installed 'KESSY installed' | Active (if equipped) | Active / Inactive | Master KESSY enable. Match to hardware. |
| KESSY: sensitivity 'KESSY sensitivity' | Medium | Low / Medium / High | Detection range for passive entry. High = activates from further away. |
| KESSY: approach unlock 'approach unlock' | Active | Active / Inactive | Car unlocks when hand approaches handle. |
| KESSY: walk-away lock 'walk away lock' | Inactive | Active / Inactive | Car locks when key fob moves out of range. |
| Push start: brake required 'brake required' | Active | Active / Inactive | Safety: brake must be pressed to start. NEVER disable for road use. |
| Push start: clutch required 'clutch required' | Active (manual only) | Active / Inactive | Safety: clutch must be pressed for manual transmission start. |
| Number of programmed keys 'key count' | Current count | Read-only | How many key fobs are programmed. Verify against actual keys held. |
| Wiper: speed-dependent interval 'speed wiper' | Active | Active / Inactive | Interval wiper speeds up with vehicle speed. |
| Turn signal resume after hazards 'signal resume' | Inactive | Active / Inactive | Last active turn signal resumes after hazards turned off. |
| Column lock: delay 'column lock delay' | 0 seconds | 0 / 5 / 10 / 30 sec | Delay before column locks after ignition off. Allows steering adjustment. |
| KESSY: interior detection zone 'interior zone' | Active | Active / Inactive | Whether key fob is detected inside car for start authorization. |
| Remote start compatibility 'remote start' | Inactive | Active / Inactive | Enable if third-party remote start module installed. |

# MODULE 16 — All Live Data Channels

| Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| KESSY: key detected | Whether key fob is in detection range | — | Detected / Not detected. |
| KESSY: key zone | Which detection zone has key | — | Driver handle / Passenger / Boot / Interior / None. |
| KESSY: key battery | Key fob battery status | — | OK / Low. Low = range reduced. |
| Key authorization: status | Whether key is authorized for start | — | Authorized / Not authorized. |
| Push button: pressed | Start button state | — | Pressed / Released. |
| Brake pedal: status (for start) | Brake pressed for start authorization | — | Pressed / Released. |
| Clutch: status (manual) | Clutch pressed for start | — | Pressed / Released. |
| Wiper stalk: position | Current wiper switch position | — | Off / Interval / Low / High / Auto. |
| Turn signal stalk: position | Current signal switch position | — | Off / Left / Right. |
| High beam stalk: position | High beam switch state | — | Off / On / Flash. |
| Column lock: state | Steering column lock | — | Locked / Unlocked. |
| Remote start: active | Whether remote start is running | — | Active / Inactive. |
| Number of invalid KESSY attempts | Failed authentication count | Count | 0 normally. Non-zero = someone tried wrong key. |

# MODULE 44 — All Basic Settings & Adaptations (Steering / EPS / LKA)

| Routine Name | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Steering angle sensor calibration 'SAS calibration' | Flat level surface. Wheels straight. Engine running. | Stores current steering position as 0°. Critical after any alignment or steering work. |
| EPS torque sensor calibration 'torque sensor calibration' | Engine running. Steering centered. No load on wheel. | Calibrates zero-torque reference for the steering torque sensor. Run after torque sensor or steering column replacement. |
| Progressive steering initialization 'progressive steering init' | If progressive steering equipped. | Initializes variable ratio steering rack position map. Run after rack replacement. |
| Lane assist calibration (dynamic) 'lane calibration' | Drive on clear marked road at 40–70 mph for 5–10 min. | Camera-based calibration for lane keeping assist. Calibration confidence increases during drive. Complete when VCDS shows sufficient confidence. |
| EPS motor test 'EPS motor test' | Vehicle stationary. Engine running. | Applies small torque pulses in each direction. Verifies motor and torque sensor respond correctly. |
| Return-to-center calibration 'return to center' | Engine running. Steering centered. | Calibrates the return-to-center spring force model. Run if steering does not return to center properly. |

# MODULE 44 — All Adaptation Channels

| Channel Name | Default | Options | Effect |
| --- | --- | --- | --- |
| LKA: installed 'LKA installed' | Active (if equipped) | Active / Inactive | Master LKA enable. Match to hardware and software. |
| LKA: steering force 'LKA force' | Medium | Low / Medium / High | Correction force applied. High = more aggressive lane centering. |
| LKA: warning mode 'LKA warning' | Vibration | Vibration / Sound / Both | How lane departure warning is communicated. |
| LKA: activation speed 'LKA speed' | 65 km/h | 60 / 65 / 70 km/h | Minimum speed for LKA function. |
| LKA: marking confidence minimum 'LKA confidence' | Standard | Low / Standard / High | How clear lane markings must be for LKA to activate. High = fewer activations. |
| TSR: display in cluster 'TSR cluster' | Active (if equipped) | Active / Inactive | Shows detected speed limit in DIS. |
| High beam assist: sensitivity 'HBA sensitivity' | Standard | Low / Standard / High | How far away oncoming lights must be to dim high beams. |
| Progressive steering: installed 'progressive steering' | Varies | Active / Inactive | Match to hardware. |
| EPS: Comfort weight 'EPS comfort' | Light | Very Light / Light / Normal | Steering effort in Comfort Drive Select mode. |
| EPS: Auto/Normal weight 'EPS normal' | Medium | Light / Medium / Firm | Effort in Auto/Normal mode. |
| EPS: Dynamic weight 'EPS dynamic' | Firm | Medium / Firm / Heavy | Effort in Dynamic mode. |
| EPS: speed-dependent gain 'EPS speed gain' | Standard | Reduced / Standard / Increased | How much heavier steering becomes at speed. |
| Steering: return-to-center force 'return force' | Medium | Light / Medium / Firm | Spring-back force when releasing steering wheel. |
| SAS calibration stored offset 'SAS offset' | 0° (after calibration) | Read-only | Should be 0° ±0.5° after correct calibration. |
| Steering wheel memory: linked to seat 'steering memory' | Inactive | Active / Inactive | If electric column equipped: steering position stored/recalled with seat memory. |

# MODULE 44 — All Live Data Channels

| Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| Steering angle: actual | Current wheel position | ° | 0° straight. Full lock: ±540° approx. |
| Steering angle: rate of change | How fast steering is turning | °/s | Fast changes during emergency maneuvers. |
| Steering torque: driver input | Force on steering wheel | Nm | Increases with steering effort. |
| Steering torque: EPS assist | Assistance torque applied | Nm | Positive = helping turn direction. |
| EPS motor: current | Motor amperage | A | Higher with more steering effort. Peak during parking. |
| EPS motor: temperature | Motor temp | °C | Normal: 40–80°C. Above 90°C = thermal reduction. |
| EPS motor: speed | Motor rotation speed | RPM | Correlates to steering speed. |
| Vehicle speed (EPS) | Speed as seen by EPS | km/h | Used for speed-sensitive assist calculation. |
| LKA: marking detected left | Left lane line detected | — | Detected / Not detected. |
| LKA: marking detected right | Right lane line detected | — | Detected / Not detected. |
| LKA: intervention active | Whether system is correcting | — | Active / Inactive. |
| LKA: correction torque | Torque LKA is applying | Nm | 0 normally. Non-zero when correcting. |
| LKA: departure direction | Which way car is drifting | — | Left / Right / Centered. |
| LKA: distance to left marking | Gap to left lane edge | m | >0.3m safe. <0.2m = LKA activates. |
| LKA: distance to right marking | Gap to right lane edge | m | Same reference. |
| TSR: current speed limit | Detected speed limit | km/h or mph | 0 = no sign detected. |
| TSR: sign type | Type of last detected sign | — | Speed limit / Zone / Highway / etc. |
| HBA: oncoming detected | Oncoming lights detected | — | Detected / Not detected. |
| HBA: leading vehicle detected | Leading lights detected | — | Detected / Not detected. |
| HBA: high beam state | Current high beam mode | — | Full on / Dimmed / Off. |
| Progressive steering: gear ratio | Current variable ratio setting | — | Changes with steering angle on progressive cars. |
| Drive Select: steering mode | Current DS-commanded weight | — | Light / Medium / Heavy. |

# MODULE 5F — All Basic Settings & Adaptations (Infotainment / MMI)

| Routine Name | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| MMI factory reset 'factory reset' | Backs up any desired presets first. | Clears all user data, presets, paired devices. Returns MMI to default state. Navigation maps not deleted. |
| Navigation map update 'map update' | USB with map update files connected. | Initiates map database update from USB. Takes 15–30 minutes. Do not interrupt. |
| Bluetooth module reset 'Bluetooth reset' | No active Bluetooth calls. | Resets Bluetooth controller. Clears all paired devices. Re-pair needed after. |
| Screen calibration 'screen calibration' | Clean screen surface. | Touch screen recalibration for accurate touch detection. |
| Radio module test 'radio test' | Good antenna signal. | Tests AM/FM/DAB reception quality. Reports signal strength per band. |
| Speaker test 'speaker test' | Volume at medium. | Cycles audio signal through each speaker individually. Identifies any non-functional speakers. |
| Amplifier calibration 'amp calibration' | After audio system modification. | Recalibrates audio output levels for speaker/amplifier combination. |
| VIM activation 'VIM' | Security Access 1804. | Enables navigation and video while vehicle is moving. No further prerequisites. |

# MODULE 5F — All Adaptation Channels

| Channel Name | Default | Options | Effect |
| --- | --- | --- | --- |
| VIM: video in motion 'VIM' | Inactive | Active / Inactive | Allows screen use while moving. Security Access 1804 required. |
| Navigation map update notify 'map notify' | Active | Active / Inactive | Disable to suppress 'maps out of date' popup. |
| Bluetooth: installed 'BT installed' | Active | Active / Inactive | Master Bluetooth enable. |
| Bluetooth: auto-connect 'BT auto connect' | Active | Active / Inactive | Auto-connects to last phone at startup. |
| Phone book: auto-download 'phone book' | Active | Active / Inactive | Disable for privacy if car is shared. |
| Maximum paired devices 'BT devices' | 10 | 5–20 | How many Bluetooth devices stored. |
| Bluetooth audio codec 'BT codec' | Standard | Standard / High / Auto | High = aptX/AAC for better audio quality. |
| CarPlay: installed 'CarPlay' | Active (if equipped) | Active / Inactive | Enable/disable Apple CarPlay. |
| Android Auto: installed 'Android Auto' | Active (if equipped) | Active / Inactive | Enable/disable Android Auto. |
| SiriusXM: installed 'SiriusXM' | Active (NA) | Active / Inactive | Must match hardware. |
| Rear camera: installed 'camera installed' | Varies | Active / Inactive | Must match hardware. |
| Parking guidelines: dynamic 'dynamic guidelines' | Active | Active / Inactive | Dynamic = lines steer with wheel. Static = fixed lines. |
| Audio: welcome sound 'welcome sound' | Inactive (most NA) | Active / Inactive | Brief chime when MMI boots. |
| Radio: startup station 'startup station' | Last station | Last / Muted | Whether radio plays on startup. |
| Screen brightness: day 'day brightness' | Auto | Auto / Level 1–10 | Daytime screen brightness. |
| Screen brightness: night 'night brightness' | Auto | Auto / Level 1–10 | Nighttime screen brightness. |
| Volume: startup level 'startup volume' | Last used | Last / Fixed level | Audio volume at startup. |
| Volume normalization 'volume normalize' | Active | Active / Inactive | Normalizes volume across sources (radio, phone, USB). |
| Speed-dependent volume 'speed volume' | Active | Active / Inactive | Volume increases with speed to compensate for road noise. |
| Audi Connect: installed 'Connect installed' | Active (if equipped) | Active / Inactive | Match to hardware. |
| Navigation: startup view 'nav startup' | Map | Map / Info / Audio | Default MMI screen at startup. |
| Voice control: hold duration 'voice hold' | 1.5 sec | 0.5 / 1 / 1.5 / 2 sec | Button hold time to activate voice control. |
| Microphone sensitivity 'mic sensitivity' | Medium | Low / Medium / High | Hands-free call microphone sensitivity. |

# MODULE 15 — All Adaptation Channels & Live Data (Airbag / SRS)

WARNING: Do not probe, modify, or test airbag circuits with any electrical tool. Work only through VCDS read functions. Outputs Tests for airbags are intentionally not listed — activating airbag squib circuits is hazardous.

## Module 15 Adaptation Channels

| Channel Name | Default | Options | Effect |
| --- | --- | --- | --- |
| Passenger airbag: installed 'passenger airbag' | Active | Active / Inactive | Must match hardware. Disable only if removing passenger airbag hardware entirely. |
| Passenger airbag: suppress threshold 'passenger threshold' | Factory (weight-based) | Adjustable kg | Weight below which passenger airbag is suppressed. Critical for child seat safety. |
| Side airbag left: installed 'side airbag L' | Active | Active / Inactive | Match to hardware. |
| Side airbag right: installed 'side airbag R' | Active | Active / Inactive | Match to hardware. |
| Knee airbag: installed 'knee airbag' | Varies by spec | Active / Inactive | Match to hardware. Not all A3 variants. |
| Pretensioner left: installed 'pretensioner L' | Active | Active / Inactive | Match to hardware. NEVER disable. |
| Pretensioner right: installed 'pretensioner R' | Active | Active / Inactive | NEVER disable. |
| Crash event data: clear 'crash data clear' | After airbag deployment | Specialist only | Module must be replaced after deployment — clearing crash data alone is not sufficient. |

## Module 15 Live Data Channels

| Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| Crash sensor: status | Current crash detection state | — | No crash / Crash detected. 'Crash detected' requires module replacement. |
| Passenger airbag: enable state | Whether passenger airbag is armed | — | Enabled / Suppressed. |
| Passenger seat occupancy | Weight sensor state | — | Occupied / Unoccupied / Child seat. |
| Passenger seat weight | Measured seat load | kg | Determines airbag deployment level. |
| Seatbelt FL: buckled | Driver seatbelt state | — | Buckled / Unbuckled. |
| Seatbelt FR: buckled | Passenger seatbelt state | — | Buckled / Unbuckled. |
| Seatbelt RL: buckled | Rear left seatbelt | — | Buckled / Unbuckled. |
| Seatbelt RR: buckled | Rear right seatbelt | — | Buckled / Unbuckled. |
| Crash sensor front left: value | Front left impact sensor signal | Raw value | Near 0 normally. Spikes during impact. |
| Crash sensor front right: value | Front right impact sensor | Raw value | Same reference. |
| Crash sensor side left: value | Left side impact sensor | Raw value | Same. |
| Crash sensor side right: value | Right side impact sensor | Raw value | Same. |
| Internal acceleration sensor X | Longitudinal acceleration in SRS module | g | 0 at rest. Changes during crash. |
| Internal acceleration sensor Y | Lateral acceleration in SRS module | g | 0 at rest. |
| Power supply: SRS module | Voltage to airbag module | V | Must be 12+ V. Low = deployment reliability reduced. |
| Reserve energy: capacitor charge | Backup capacitor state | V or % | Full charge maintained. Low = SRS cannot deploy if battery lost in crash. |
| Active faults count | Number of current SRS faults | Count | 0 in healthy system. Any fault = service required. |

# MODULE 19 — All Adaptations & Live Data (CAN Gateway)

## Module 19 Adaptation Channels

| Channel Name | Default | Options | Effect |
| --- | --- | --- | --- |
| Module installation list byte 0 'module byte 0' | Varies per car | Bit field | Each bit = one module installed. Do not modify manually — use AutoScan to manage. |
| Module installation bytes 1–15 'module bytes' | Varies | Bit fields | Additional module presence bits. Extensive — managed by VCDS AutoScan and coding helper. |
| CAN bus: high speed enabled 'CAN HS' | Active | Active / Inactive | High speed CAN bus enable. NEVER disable. |
| CAN bus: medium speed enabled 'CAN MS' | Active | Active / Inactive | Medium speed CAN bus. NEVER disable. |
| Diagnostic interface: enabled 'OBD interface' | Active | Active / Inactive | OBD-II diagnostic port. NEVER disable. |
| Market: regional configuration 'market code' | North America (CZPB) | Various region codes | Affects certain gateway behaviors. Match to actual vehicle market. |
| VIN: stored 'VIN' | Vehicle VIN | Read-only via this channel | Gateway stores VIN for module network verification. |

## Module 19 Live Data Channels

| Channel | Description | Unit | Normal / Notes |
| --- | --- | --- | --- |
| CAN HS: bus status | High speed CAN health | — | Active / Fault. Fault = major network issue. |
| CAN MS: bus status | Medium speed CAN health | — | Active / Fault. |
| Number of modules online | How many modules gateway sees | Count | Should match number in AutoScan. |
| Module 01: communication | Engine module CAN status | — | Online / Offline. |
| Module 02: communication | Transmission CAN status | — | Online / Offline. |
| Module 03: communication | Brake CAN status | — | Online / Offline. |
| Module 09: communication | Central electronics status | — | Online / Offline. |
| Module 17: communication | Cluster CAN status | — | Online / Offline. |
| Module 44: communication | Steering CAN status | — | Online / Offline. |
| Gateway: internal temperature | Gateway module temp | °C | 40–70°C normal. |
| Gateway: supply voltage | Power at gateway module | V | 12.4–14.4V normal. |
| OBD: session active | Whether diagnostic session open | — | Active (when VCDS connected). |
| Ignition: status via gateway | Current ignition state | — | Off / Accessory / On / Start. |

# MODULE 65 — All Basic Settings & Adaptations (Direct TPMS)

| Routine Name | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| TPMS sensor registration 'sensor registration' | All four sensors installed in wheels at correct pressure. | Associates each sensor ID with its wheel position (FL, FR, RL, RR). Drive 5+ min after to confirm. |
| TPMS reference pressure store 'store pressures' | All tires at correct cold inflation. | Stores current pressures as reference baseline. Same as Module 17 TPMS calibration but from the receiver module directly. |
| Sensor battery check 'sensor battery test' | All sensors transmitting. | Polls each sensor for battery voltage status. Identifies sensors with low batteries before they fail. |
| Sensor wakeup test 'sensor wakeup' | TPMS tool or VCDS output. | Sends RF wakeup signal to each sensor. Verifies all four are responding. Non-responding sensor = dead battery or sensor fault. |

# MODULE 65 — All Adaptation Channels

| Channel Name | Default | Options | Effect |
| --- | --- | --- | --- |
| TPMS: installed 'TPMS installed' | Active (if direct TPMS) | Active / Inactive | Master TPMS enable. |
| Sensor FL: ID 'sensor FL ID' | Unique hex code | Enter new sensor ID | Enter new sensor ID after replacing FL wheel sensor. |
| Sensor FR: ID 'sensor FR ID' | Unique hex code | Enter new sensor ID | Same for FR. |
| Sensor RL: ID 'sensor RL ID' | Unique hex code | Enter new sensor ID | Same for RL. |
| Sensor RR: ID 'sensor RR ID' | Unique hex code | Enter new sensor ID | Same for RR. |
| Warning threshold: low pressure 'low threshold' | Typically -0.3 bar from target | Adjustable | How much below target pressure triggers warning. |
| Warning threshold: critical pressure 'critical threshold' | Typically -0.6 bar from target | Adjustable | Critical low pressure threshold for urgent warning. |
| Temperature compensation: active 'temp compensation' | Active | Active / Inactive | Compensates displayed pressure for tire temperature. Keep Active. |

# MODULE 69 — All Basic Settings & Adaptations (Trailer Recognition)

| Routine Name | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Trailer light test 'trailer light test' | Trailer connected. | Cycles all trailer light outputs (tail, brake, turn L, turn R). Verifies all trailer socket pins functioning. |
| Trailer recognition calibration 'trailer calibration' | With and without trailer. | Learns normal current draw with and without trailer for reliable trailer detection. |

# MODULE 69 — All Adaptation Channels

| Channel Name | Default | Options | Effect |
| --- | --- | --- | --- |
| Trailer: installed 'trailer installed' | Inactive | Active / Inactive | Enable when tow hitch physically fitted. |
| Connector: type 'connector type' | 7-pin | 7-pin / 13-pin | Match to installed connector. |
| Lighting check: active 'lighting check' | Active | Active / Inactive | Monitors trailer lights for blown bulbs. |
| ESP: trailer mode 'trailer ESP' | Active | Active / Inactive | Adjusts ESP for towing. Leave Active. |
| Trailer stability 'trailer sway' | Active | Active / Inactive | Sway detection and correction. Leave Active when towing. |
| Speed limit when towing 'tow speed limit' | 130 km/h | 80 / 100 / 120 / 130 km/h | Warning threshold when trailer connected. |
| Maximum permitted tongue weight 'tongue weight' | 80 kg | Read-only reference | Physical limit — do not exceed. |
| Retractable hitch: installed 'retractable hitch' | Varies | Active / Inactive | Enable if motorized retractable hitch is fitted. |
| Hitch deployed: position stored 'hitch position' | Learned automatically | Reset | Recalibrate if hitch does not fully deploy or retract. |

— End of Exhaustive BS / Adaptations / Live Data — Volume 2 —

Modules covered: 13, Climate, 61, 36, 16, 44, 5F, 15, 19, 65, 69 — Complete channel-level reference