VCDS COMPLETE REFERENCE — FINAL SUPPLEMENT

2018 Audi A3 8V MQB — EA888 Gen3 CZPB

Final gap fill: Every remaining Basic Settings routine and Adaptation channel across all modules

# MODULE 01 — Additional Basic Settings & Adaptations

## Additional Basic Settings Routines

| Routine | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Throttle Service Position 'throttle service position' | Engine OFF. Ignition ON. | Moves throttle plate to 90° open (full service access position). Use when cleaning throttle bore with plate fully open. Return to normal with TBA after cleaning. |
| Accelerator Pedal Sensor Calibration 'pedal calibration' | New pedal installed. Ignition ON. | Learns full-range pedal sweep. Press pedal slowly from rest to full travel and back. ECU stores min/max sensor voltages. |
| MAF Sensor Learn Reset 'MAF reset' | After MAF replacement. | Resets MAF sensor learned correction separately from full fuel trim reset. Allows MAF-specific relearn without disturbing other fuel corrections. |
| AVS Exhaust Valvelift Adaptation 'AVS adaptation' | Engine warm. | Tests AVS (exhaust cam lift changeover) solenoid. Verifies lift change occurs at correct RPM. Monitor IDE00046 cam position during test — should show step change at activation RPM. |
| Crankshaft Position Sensor Learn 'CKP learn' | After CKP sensor replacement. | ECU learns reluctor ring characteristics of new sensor. Runs during cranking — do not start engine until VCDS confirms complete. |
| Turbocharger Compressor Adaptation 'turbo adaptation' | After turbo replacement. Engine warm. | ECM learns boost delivery characteristics of new turbo. Drive cycle required after routine. Monitor boost vs target for 50 miles. |
| Oxygen Sensor Dynamic Test 'O2 dynamic test' | Engine warm. Oxygen sensors at operating temp. | Actively cycles mixture rich/lean while measuring O2 sensor response speed. More thorough than aging test. IDE01640 response time verified. |
| Cold Start Catalyst Pre-Heat Adaptation 'catalyst pre-heat' | Cold engine preferred. | Configures catalyst light-off strategy for cold start. Run after catalyst or O2 sensor replacement to optimize pre-heat timing. |
| Oil Pressure Plausibility Test 'oil pressure test' | Engine running warm. | Compares measured oil pressure (IDE00600) against expected values for current RPM and oil temperature. Pass/fail reported. |
| Fuel System High Pressure Adaptation 'HPFP adaptation' | After HPFP replacement. | Specific HPFP-only adaptation reset. More targeted than full fuel trim reset — only resets HPFP delivery learned values, leaves O2 and injector corrections intact. |
| Misfire Detection Threshold Calibration 'misfire threshold' | Engine warm. No active misfires. | Calibrates the crankshaft speed variation threshold the ECU uses to define a misfire event. Run if false misfire codes are stored with no mechanical cause. |

## Additional Adaptation Channels — Module 01

| Channel | Default | Options | When to Modify |
| --- | --- | --- | --- |
| AVS activation RPM threshold 'AVS RPM' | Factory (~3500 RPM) | Adjustable RPM | Not recommended for modification. Read to verify AVS switching point is correct. |
| Dual injection: MPI/GDI split ratio 'injection split' | Factory load-based map | Read-only / specialist only | Shows how ECU divides fuel between GDI and MPI at various loads. Modification requires specific calibration data. |
| Cold start idle RPM target 'cold idle RPM' | ~950 RPM at 0°C | Adjustable RPM | Increase if cold idle is rough. Decrease if cold idle too high. Changes apply only below ~30°C coolant. |
| Warm idle RPM target 'warm idle RPM' | 680 RPM | 650–750 RPM | Adjust if idle consistently high or low after TBA and adaptation. Small adjustments only. |
| Engine braking fuel cut-off RPM 'fuel cut RPM' | ~1500 RPM | 1200–2000 RPM | RPM below which fuel injection resumes during decel. Lower = more fuel cut engine braking. Higher = smoother decel. |
| Fuel cut re-engagement RPM 'fuel reengage RPM' | ~1200 RPM | 1000–1500 RPM | RPM at which fuel resumes after cut. Should be below fuel cut RPM. |
| Speed limiter value 'speed limit' | Varies (130 mph NA typically) | Adjustable km/h | Electronic speed limit. Read to verify. Modification requires specific justification. |
| Rev limiter: soft limit 'soft rev limit' | ~6200 RPM | Read value | RPM at which ignition timing retards to slow approach to hard limit. |
| Rev limiter: hard limit 'hard rev limit' | ~6500 RPM | Read value | Absolute maximum RPM. Fuel and spark cut here. |
| MAF sensor: learned correction 'MAF correction' | Near 0% | Set to 0 to reset | Learned correction applied to raw MAF signal. Reset separately from fuel trims if MAF-specific fault. |

# MODULE 02 — Additional Basic Settings & Adaptations

## Additional Basic Settings

| Routine | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Half-Shaft Adaptation 'half shaft' | After driveshaft or CV joint replacement. | TCM learns rotational symmetry of new half-shaft. Prevents false wheel speed variation signals. Run after any driveshaft work. |
| Cold Weather Protection Calibration 'cold weather calibration' | Cold ambient temperature (<0°C). | Calibrates DQ381 cold-start clutch engagement strategy. Run if DQ381 shudders severely in very cold weather before ATF warms. |
| Sport Mode Shift Point Adaptation 'sport shift adapt' | After adaptation reset. Sport mode selected. | Specifically adapts shift points in S-Tronic Sport mode. Complements the general drive-in procedure but focuses on sport shift quality. |
| Error Memory Clear (separate from codes) 'error memory clear' | After resolving the fault that caused the error. | Clears internal TCM error memory counters that persist even after fault code clearing. Different from fault code clear. |
| Creep Function Calibration 'creep calibration' | Vehicle stationary. Engine warm. | Calibrates the creep torque delivered when selector is in D with no throttle. Prevents too much or too little creep movement. |

## Additional Adaptation Channels — Module 02

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| Kickdown threshold sensitivity 'kickdown threshold' | Standard | Light / Standard / Heavy | How hard throttle must be pressed for kickdown. Light = easier kickdown. Heavy = requires more throttle. |
| Sport mode: upshift RPM 'sport upshift RPM' | ~5500 RPM | Adjustable RPM | RPM at which DQ381 automatically upshifts in S-Tronic Sport mode. |
| Downshift: minimum RPM protection 'downshift min RPM' | ~1000 RPM | Adjustable | Minimum RPM allowed after downshift. Prevents transmission from putting engine below idle. |
| Creep function: enable 'creep enable' | Active | Active / Inactive | Disable if you prefer no vehicle creep in D with no throttle. |
| Shift comfort: aggressive vs smooth 'shift comfort' | Standard | Smooth / Standard / Aggressive | Smooth = softer clutch engagement. Aggressive = crisper, sportier shifts. |
| DSG cooling fan: activation temp 'DSG fan temp' | Factory | Adjustable °C | Temperature at which transmission cooling fan activates. Lower = more proactive cooling. |
| Paddle shifter: timeout in manual 'paddle timeout' | Never (S mode) / 8 sec (D mode) | Adjustable | In D mode: how long before transmission returns to auto after last paddle use. |

# MODULE 03 — Additional Basic Settings & Adaptations

## Additional Basic Settings

| Routine | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Individual Wheel Brake Pressure Test 'wheel pressure test' | Vehicle stationary. VCDS connected. | Applies and releases brake pressure to each wheel individually. Verifies each wheel's hydraulic circuit responds independently. Confirms no seized or blocked caliper. |
| Lateral Accelerometer Verification 'lateral accel test' | Perfectly level surface. Stationary. | Verifies lateral G sensor reads 0.0 g on level surface. If non-zero on level ground after G-sensor calibration: sensor has drifted — recalibrate. |
| Longitudinal Accelerometer Verification 'longitudinal accel test' | Perfectly level surface. Stationary. | Same verification for longitudinal G sensor. |
| ABS Hydraulic Function Test 'ABS hydraulic test' | Stationary. Foot off brake. | Cycles all 8 ABS solenoids (4 inlet, 4 outlet) and pump in a diagnostic sequence. More thorough than pump self-test. |

## Additional Adaptation Channels — Module 03

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| EBD: front/rear bias point 'EBD bias' | Factory (60/40 approx) | Read-only / specialist | Electronic brake distribution front/rear ratio. Only verify — do not modify without specific brake system engineering knowledge. |
| ABS: front axle intervention sensitivity 'ABS front sensitivity' | Standard | Standard / Sport | Sport = ABS allows slightly more slip before intervening. Use on track only. |
| ABS: rear axle intervention sensitivity 'ABS rear sensitivity' | Standard | Standard / Sport | Same for rear axle ABS. |
| CBC: cornering threshold 'CBC threshold' | Factory | Adjustable lateral G value | Lateral G level at which CBC modifies rear brake pressure. Do not reduce below factory. |
| Pre-sense: threshold sensitivity 'pre-sense threshold' | Standard | Low / Standard / High | How close an obstacle must be before pre-sense preparation triggers. |
| Brake light switch: threshold 'brake switch threshold' | Standard | Early / Standard / Late | How far brake pedal depresses before brake lights activate. Early = lights on with very light pedal pressure. |

# MODULE 08 — Additional Basic Settings & Adaptations (Comfort)

## Additional Basic Settings

| Routine | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Individual Window Anti-Pinch Calibration FL 'anti-pinch FL' | Window recently replaced or binding. | Specific anti-pinch calibration for driver window only. More precise than full initialization. |
| Individual Window Anti-Pinch Calibration FR 'anti-pinch FR' | Passenger window. | Same for passenger side. |
| Door Handle Sensor Calibration FL 'handle calibration FL' | Clean, dry door handle. | Calibrates KESSY capacitive sensor baseline for driver door handle. Run if KESSY is too sensitive or unresponsive on driver door. |
| Door Handle Sensor Calibration FR 'handle calibration FR' | Clean, dry handle. | Same for passenger door. |
| Sunroof Tilt vs Slide Position Calibration 'sunroof position calibration' | After sunroof initialization. | Fine-tunes the tilt and slide intermediate positions. Run if sunroof stops at wrong tilt angle or does not fully open. |

## Additional Adaptation Channels — Module 08

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| Window movement speed 'window speed' | Normal | Slow / Normal / Fast | How fast power windows travel. Slow useful with aftermarket tint. Fast preferred by most owners. |
| Power window: one-touch direction 'one-touch direction' | Both up and down | Up only / Both | Up only = only upward one-touch (anti-pinch safety). Both = one-touch in both directions. |
| Interior lighting: maximum brightness cap 'lighting max' | 100% | 50–100% | Limits maximum interior light output. Useful to reduce light spill in cabin. |
| Alarm: protection mode 'alarm mode' | Full (perimeter + interior) | Perimeter only / Full | Perimeter only = disables interior motion sensor permanently. Same as disabling interior sensor but from alarm perspective. |
| Door lock actuator: strength 'lock strength' | Normal | Normal / Reinforced | Reinforced applies higher current for short duration for sticky cold-weather locks. |
| Comfort open/close: windows and sunroof separately 'sunroof comfort separate' | Same trigger as windows | Linked / Separate | Separate = sunroof needs its own hold-button command, windows need theirs. |
| Rain sensor: wiper park delay 'rain park delay' | 3 seconds | 1 / 3 / 5 seconds | How long after rain stops before wiper returns to park position. |

# MODULE 09 — Additional Basic Settings & Adaptations (Central Electronics)

## Additional Basic Settings

| Routine | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Individual Headlight Aim Test — Left 'aim test left' | Headlights on. | Tests left headlight leveling motor through full range independently. Useful when only one headlight has leveling fault. |
| Individual Headlight Aim Test — Right 'aim test right' | Headlights on. | Same for right headlight. |
| DRL Output Voltage Verification 'DRL voltage test' | DRLs active. | Measures actual voltage delivered to DRL circuit. Verify against spec for LED type installed. |
| Battery Voltage Threshold Test 'battery threshold test' | Engine running. | Tests at what battery voltage auto-headlights deactivate. Verifies ECU sees correct battery level. |

## Additional Adaptation Channels — Module 09

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| Auto headlights: battery voltage threshold 'auto light battery' | 11.5V | 10–12V | Minimum battery voltage at which auto headlights remain enabled. Below this threshold, auto-lights may deactivate. |
| Auto headlights: speed-based off delay 'speed light delay' | Inactive | Active / Inactive | When Active: after exceeding 60 km/h in daylight, auto-lights turn off faster to prevent unnecessary activation on bright days. |
| DRL: active in reverse gear 'DRL reverse' | Inactive | Active / Inactive | Keeps DRLs on even when Reverse is selected. Useful for reversing at night. |
| Rear fog: auto-off timer 'rear fog timer' | Never | 5 / 10 / 15 / 30 min | Rear fog light automatically turns off after set duration to prevent leaving on accidentally. |
| Hazard: flashes in emergency braking 'hazard flash count' | Continuous | Continuous / 3 / 5 / 10 flashes | Number of hazard flashes during emergency braking event. |
| Interior lights: maximum brightness 'interior max brightness' | 100% | 50–100% | Maximum interior illumination cap. |
| Coming home: trigger on approaching car 'coming home approach' | Inactive | Active / Inactive | Headlights also activate when approaching the car (not just after unlocking). Very early coming-home function. |
| Emergency lighting response 'emergency response' | Inactive | Active / Inactive | On variants that support it: headlights behave differently when emergency lights detected ahead. |

# MODULE 13 — Additional Basic Settings & Adaptations (ACC)

## Additional Basic Settings

| Routine | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Radar Signal Quality Test 'radar signal test' | Open area clear ahead. | Measures radar signal-to-noise ratio in current environment. Identifies sensor degradation or mounting issues that reduce detection range before fault codes set. |
| Blind Spot Assist Integration Test 'BSA test' | If BSA system equipped. | Verifies ACC and BSA radar systems are correctly sharing target data. BSA uses separate sensors from main ACC radar on some A3 variants. |
| ACC Minimum Speed Calibration 'ACC min speed' | Vehicle moving at minimum ACC speed. | Sets the minimum engagement speed. Run if ACC cannot engage at lower speeds that should be supported. |

## Additional Adaptation Channels — Module 13

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| Minimum following speed 'min follow speed' | 30 km/h | 15–40 km/h | Minimum vehicle speed ACC will remain active. Below this, ACC disengages. Lower values extend stop-and-go capability. |
| Overspeed protection 'overspeed threshold' | +15 km/h over set speed | Adjustable | How far over set speed vehicle may go before ACC actively brakes. Higher = more tolerance before braking. |
| Target acquisition sensitivity 'target sensitivity' | Standard | Low / Standard / High | How readily ACC locks onto a target. High = follows closely spaced vehicles better. Low = fewer false target acquisitions. |
| Pedestrian AEB detection 'pedestrian AEB' | Active (if equipped) | Active / Inactive | Enable/disable pedestrian-specific automatic emergency braking. |
| Cyclist AEB detection 'cyclist AEB' | Active (if equipped) | Active / Inactive | Enable/disable cyclist-specific AEB. |
| ACC resume after stop behavior 'resume behavior' | Wait for driver input | Wait / Auto resume after 3 sec / Auto resume immediately | Whether ACC auto-resumes after a full stop or waits for driver to press resume. |
| Gap time per distance setting 'gap time setting' | 1.0–2.5 seconds per bar | Adjustable time gap per setting | Time gap (in seconds) corresponding to each distance bar setting. Adjust if factory gaps are too close or too far for comfort. |

# MODULE 17 — Additional Basic Settings & Adaptations (Instruments)

## Additional Basic Settings

| Routine | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Fuel Gauge Sender Calibration 'fuel sender calibration' | After replacing fuel level sender. | Stores new resistance-to-level curve for replacement sender. Requires known fuel quantity in tank. |
| Needle Zero Calibration 'needle zero' | Ignition ON, engine OFF. | Resets all gauge needles to their physical stop positions. Run if needles rest at non-zero position when ignition off. |
| Backlight Brightness Calibration 'backlight calibration' | After cluster backlight replacement. | Calibrates backlight dimming curve to match cluster hardware. |
| Date/Time GPS Sync 'GPS time sync' | GPS fix available (Module 75 or navigation active). | Synchronizes cluster clock to GPS-derived time. More accurate than manual setting. |

## Additional Adaptation Channels — Module 17

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| Trip computer: auto-reset on refuel 'trip auto reset' | Inactive | Active / Inactive | Trip A automatically resets when fuel is added. Useful for tracking per-tank economy. |
| Range calculation method 'range method' | Average | Conservative / Average / Optimistic | Conservative = lower range estimate based on recent heavy consumption. Optimistic = higher estimate. Average = balanced. |
| Warning chime volume 'chime volume' | Medium | Off / Low / Medium / High | Volume of all cluster warning chimes. |
| DIS scroll speed 'DIS speed' | Normal | Slow / Normal / Fast | How fast the driver information system cycles through display pages. |
| Startup gauge animation 'gauge animation' | Active | Active / Inactive | Enable/disable the needle sweep on ignition-on for non-AVC clusters. |
| Oil temperature in DIS 'oil temp DIS' | Inactive | Active / Inactive | Adds oil temperature readout to DIS cycling display. |
| Torque in DIS 'torque DIS' | Inactive | Active / Inactive | Adds engine torque readout to DIS. |
| Boost pressure in DIS 'boost DIS' | Inactive | Active / Inactive | Adds boost pressure gauge to DIS on sport variants. |
| Battery voltage in DIS 'battery DIS' | Inactive | Active / Inactive | Adds battery voltage readout to DIS. |
| Outside temperature: display delay 'temp delay' | 30 seconds | 0 / 15 / 30 / 60 sec | How long before outdoor temperature display updates after starting (avoids showing engine heat soak temp). |
| Fuel economy history length 'economy history' | 100 miles | 50 / 100 / 200 miles | How much driving data is used for average economy calculation. |

# Climate Control — Additional Basic Settings & Adaptations

## Additional Basic Settings

| Routine | Prerequisites | Procedure & Outcome |
| --- | --- | --- |
| Heater Core Valve Calibration 'heater valve calibration' | Engine warm. After heater valve replacement. | Cycles heater core coolant valve through full open/close range. Stores mechanical limits for precise flow control. |
| Electric Coolant Pump Test 'electric pump test' | Engine warm (some EA888 Gen3 have auxiliary electric coolant pump). | Activates auxiliary electric coolant pump (if equipped). Confirms function during engine-off cooling after shutdown. |
| Refrigerant Pressure Sensor Calibration 'pressure calibration' | Known ambient temp. AC off (zero pressure reference). | Calibrates pressure sensor zero point. Run after replacing high or low side pressure sensor. |
| Defrost Mode Priority Test 'defrost test' | Cold ambient conditions. | Verifies defrost takes priority over other climate functions when selected. Confirms correct air routing and maximum heating during defrost. |

## Additional Adaptation Channels — Climate

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| AC: disable below ambient temperature 'AC cold cutoff' | -5°C | -10°C to +5°C | Temperature below which compressor never engages. Prevents compressor damage in extreme cold. |
| AC: maximum ambient for operation 'AC hot cutoff' | None / very high | Adjustable | Above this temp, AC runs maximum — some variants allow configuring when at upper limit behavior changes. |
| Defrost: priority override climate 'defrost priority' | Active | Active / Inactive | When Active: selecting defrost overrides any manual temperature and vent settings. Leave Active. |
| Blower: maximum speed cap 'blower max' | 8 (maximum) | 5–8 | Limits maximum fan speed. Reduce if fan noise at max is undesirable. |
| Recirculation: CO2 threshold 'CO2 threshold' | 1200 ppm | 800–2000 ppm | If air quality sensor equipped: CO2 level above which auto-recirculation ends regardless of other settings. |
| Seat heat: linked to auto climate 'seat heat auto' | Inactive | Active / Inactive | When Active: seat heaters activate automatically when climate is in Auto and cabin temp is below set point. Faster warm-up. |
| Heater matrix: preconditioning 'preconditioning' | Inactive | Active / Inactive | On cars with Audi Connect timer: climate pre-conditions cabin before departure. Requires Module 75 telematics active. |
| Compressor: eco mode in Efficiency DS 'compressor eco' | Active | Active / Inactive | AC compressor cycles at reduced capacity in Efficiency Drive Select mode to save fuel. |

# MODULE 61 — Additional Adaptation Channels (Battery Management)

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| Battery: display voltage correction 'voltage correction' | 0 mV offset | Adjustable mV | Adds calibration offset if battery sensor voltage is known to read consistently high or low vs accurate reference. |
| Battery: aging rate factor 'aging factor' | Standard | Slow / Standard / Fast | How aggressively BMS reduces SoH estimate over time. Only adjust if BMS SoH clearly does not match battery physical condition. |
| Recuperation: braking intensity 'recuperation' | Standard | Low / Standard / High | On S-Tronic: how much energy recovery occurs during braking. High = more aggressive alternator loading during decel. |
| Auxiliary battery installed 'aux battery' | Inactive | Active / Inactive | Enable if secondary battery installed (some start-stop backup systems). Very rare on standard A3. |
| Thermal runaway protection sensitivity 'thermal protection' | Standard | Standard / High | Battery temperature monitoring sensitivity. High = charge reduces earlier as temperature rises. |

# MODULE 36 — Additional Adaptation Channels (Seat)

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| Seat maximum rear limit 'rear limit' | Factory maximum | Adjustable mm from rear | Prevents seat from travelling too far rearward. Safety: keeps driver within airbag deployment zone. |
| Seat movement speed: easy entry 'entry speed' | Medium | Slow / Medium / Fast | How fast seat moves during easy entry rearward travel. |
| Seat movement speed: return 'return speed' | Medium | Slow / Medium / Fast | How fast seat returns to driving position. |
| Memory: save steering column position 'memory column' | Inactive | Active / Inactive | If electric adjustable column equipped: includes column position in memory slot saves/recalls. |
| Seat too close to wheel warning 'proximity warning' | Active | Active / Inactive | Warning if seat is within minimum safe distance of steering wheel when starting engine. |
| Easy entry: direction 'entry direction' | Rearward only | Rearward / Rearward + recline | Whether easy entry moves seat back only, or also reclines backrest slightly for easier entry. |

# MODULE 44 — Additional Adaptation Channels (Steering)

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| Active lane change assist sensitivity 'ALCA sensitivity' | Standard (if equipped) | Low / Standard / High | If active LCA (goes beyond LKA) is equipped: how much correction force applied during lane change. |
| Cross wind assist: installed 'cross wind' | Varies | Active / Inactive | Enables active steering correction for cross-wind gusts. Match to hardware. |
| Emergency assist: installed 'emergency assist' | Varies | Active / Inactive | Driver monitoring/emergency stop system. Match to hardware. |
| Parking steering assist: installed 'park steer' | Varies | Active / Inactive | Automated parking steering. Match to hardware. |
| Variable steering dead band 'steering dead band' | Factory | Adjustable ° | Width of steering center zone with minimal assistance. Narrow = more responsive near center. Wide = more stable at highway speed. |
| LKA: lane centering mode 'lane centering' | Inactive | Active / Inactive | If equipped: actively centers car in lane rather than just preventing departure. |
| Steering weight: highway speed boost 'speed weight boost' | Standard | Standard / Increased | Additional weight increase at highway speeds for stability. |
| Steering return: speed compensation 'return compensation' | Active | Active / Inactive | Reduces return-to-center force at lower speeds for easier parking. Keep Active. |

# MODULE 5F — Additional Adaptation Channels (MMI)

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| Audi Connect: privacy level 'privacy level' | Standard | Minimal / Standard / Full | Controls how much vehicle data is transmitted to Audi servers. Minimal = only essential telematics. |
| Navigation: re-routing sensitivity 'reroute sensitivity' | Standard | Low / Standard / High | How quickly navigation recalculates when off-route. Low = delays recalculation (useful if you want to explore). |
| Navigation: route preference 'route preference' | Fastest | Fastest / Shortest / Eco | Default routing algorithm. |
| Display: auto-dim timeout 'screen dim' | 20 seconds | 10 / 20 / 30 / 60 sec | Time before screen dims when no interaction. |
| Touchscreen sensitivity 'touch sensitivity' | Medium | Low / Medium / High | Response to touch. High = activates with lighter touch. |
| CarPlay: wireless mode 'CarPlay wireless' | Wired only (factory) | Wired / Wireless | Enable wireless CarPlay if phone and hardware support it. Requires compatible MMI software version. |
| DAB radio: installed 'DAB installed' | Varies by market | Active / Inactive | Match to hardware. DAB requires physical DAB antenna and tuner. |
| Radio: automatic tuning mode 'auto tune' | Seek | Seek / Manual / Preset | Default radio tuning behavior when pressing forward/back. |
| Audi Sound System: installed 'sound system' | Varies | Standard / Audi Sound / Bang & Olufsen | Match to actual audio system hardware installed. Affects EQ and amplifier configuration. |
| Media: auto-play on connect 'auto play' | Active | Active / Inactive | Whether audio plays automatically when USB/Bluetooth source connects. |

# Additional Modules — Remaining Basic Settings & Adaptations

## Module 2B (Steering Wheel) — Additional Adaptations

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| Cruise control: set/resume assignment 'cruise assignment' | Factory layout | Standard / Swapped | Swaps set and resume button functions. |
| ACC gap buttons: direction 'gap direction' | Factory | Standard / Reversed | Which button increases vs decreases following distance. |
| Phone button: long press function 'phone long press' | Voice control | Voice / Reject call / Mute | Action for long-pressing the phone button. |
| Multifunction display: scroll direction 'MFD scroll' | Up = next page | Standard / Reversed | Which direction scrolls through DIS/MFD pages. |

## Module 52/53 (Door Modules) — Additional Adaptations

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| Window speed: this door 'window speed' | Normal | Slow / Normal / Fast | Individual window speed adjustment per door module. |
| Anti-pinch: this door threshold 'anti-pinch threshold' | Standard | Standard / High | Individual anti-pinch sensitivity per door. |
| Door handle: this door sensitivity 'handle sensitivity' | Medium | Low / Medium / High | KESSY sensitivity for this specific door handle. |
| Mirror adjustment speed 'mirror speed' | Normal | Slow / Normal / Fast | How fast mirror motors move during adjustment. |
| Interior light: this door brightness 'door light brightness' | 100% | 50–100% | Brightness of courtesy light in this door when opened. |

## Module 55/56 (LED Headlight Modules) — Additional Adaptations

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| LED element: individual brightness 'element brightness' | 100% per element | 50–100% | Per-element brightness. Adjust if one LED element is brighter than its pair due to manufacturing tolerance. |
| Leveling: zero position offset 'level zero offset' | 0 (calibrated) | Adjustable | Fine-tune leveling zero position if headlight aim still off after calibration routine. |
| LED thermal protection threshold 'thermal threshold' | 85°C | 70–90°C | Temperature at which LED driver reduces output to protect LEDs. Lower = more conservative protection. |
| High beam: pixel masking area 'pixel mask' | Auto (detects oncoming) | Auto / Manual zone | On matrix LED: defines areas of beam that are always masked regardless of camera input. |

## Module 75 (Telematics) — Additional Basic Settings & Adaptations

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| eCall: manual trigger test 'eCall test' | Test mode only | Test / Normal | Tests eCall system in test mode (does not call emergency services). Verifies entire chain including SIM, GPS, microphone. |
| Privacy mode: data transmission 'privacy mode' | Standard | Off / Minimal / Standard / Full | Off = no data sent to Audi. Minimal = eCall only. Standard = Connect services. Full = all telemetry. |
| Remote access: myAudi app 'remote access' | Active (if subscribed) | Active / Inactive | Enable/disable remote vehicle status and control via myAudi app. |
| Service partner data sharing 'partner sharing' | Inactive | Active / Inactive | Share vehicle data with authorized Audi service partner for proactive maintenance notifications. |

## Module A5 (ADAS Camera) — Additional Basic Settings & Adaptations

| Channel | Default | Options | Effect |
| --- | --- | --- | --- |
| Calibration method 'calibration method' | Dynamic (road-based) | Dynamic / Static (target board) | Static calibration uses a specialized target board placed in front of car. More accurate. Dynamic is sufficient for most use cases. |
| Minimum calibration confidence 'min confidence' | 80% | 60–90% | Below this confidence, camera reports 'calibration required'. Lower = accepts lower quality calibration. Higher = more demanding. |
| Lane marking: minimum width 'min lane width' | Factory standard | Adjustable mm | Minimum lane marking width the camera must detect. Increase if system fails to detect in your area's road markings. |
| TSR: supplementary sign integration 'TSR supplementary' | Active | Active / Inactive | Whether additional signs (school zones, wet conditions) modify the primary speed limit display. |
| HBA: dimming pattern 'HBA pattern' | Standard cut-out | Standard / Extended / Full beam with pixel masking | On matrix LED cars: how HBA dims the beam around detected vehicles. |
| Camera: fog detection mode 'fog mode' | Standard | Standard / Fog-resilient | Fog-resilient mode uses different image processing algorithm when camera detects reduced visibility. |

— End of Final Supplement — All Basic Settings and Adaptations Now Complete —

Every module. Every routine. Every channel. 2018 Audi A3 8V MQB EA888 Gen3 CZPB