VCDS COMPLETE REFERENCE MANUAL

2018 Audi A3 8V MQB — 2.0 TSI EA888 Gen3 (CZPB)

Module 13: Distance Regulation — Adaptive Cruise Control (ACC) / Radar Sensor

# Section 1: System Identification

Module 13 is the Distance Regulation module — this is the Adaptive Cruise Control (ACC) system and its associated forward radar sensor. The radar sensor (designated J428 in Audi documentation) is a 77 GHz forward-facing radar unit mounted behind the front bumper. It measures distance and relative velocity to vehicles ahead and manages cruise speed accordingly.

This module is directly relevant to a known issue: disconnecting the ACC sensor connector while the vehicle is powered results in a misalignment fault that locks the module and prevents calibration until a specific unlock and recalibration sequence is performed using VCDS.

## Module 13 Identification

| Specification | Value |
| --- | --- |
| Module number in VCDS | 13 |
| Module name | Distance Regulation, ACC, Adaptive Cruise, J428 |
| Sensor designation | J428 — ACC distance sensor / radar sensor |
| Radar frequency | 77 GHz millimeter-wave radar |
| Radar range | Approximately 150 meters forward detection range at highway speeds |
| Sensor location | Behind front bumper cover, center-mounted. On A3 8V: behind the lower front grille area, typically center or slightly offset. |
| Protocol | UDS/CAN — named dropdown channels in VCDS |
| Systems integrated with | Module 01 (engine — throttle control), Module 02 (transmission — gear selection during ACC), Module 03 (ABS — braking during ACC), Module 44 (steering — lane keeping assist coordination on equipped cars) |
| Features provided | ACC (adaptive cruise), Front Assist (collision warning), Autonomous Emergency Braking (AEB) on equipped cars |
| Security access code | No security access required for calibration Basic Settings on this module |

IMPORTANT — KNOWN ISSUE WITH THIS MODULE: Disconnecting the J428 connector while the vehicle is powered (ignition ON or engine running) causes the module to store a hard misalignment fault. This fault prevents the module from accepting calibration commands until the fault-unlock and recalibration sequence described in Section 4 is performed. This is not a hardware failure — it is a software lock that VCDS can resolve. This is the most common reason ACC is disabled after bumper or wiring work.

# Section 2: Accessing Module 13

## Access Procedure

- HEX-V2 connected. VCDS launched. Ignition ON, engine running preferred for calibration.

- Click [Select Control Module] → [13 — Distance Regulation].

- VCDS connects to J428 module. May take 4–8 seconds — UDS modules with active faults can be slow to respond.

- Module window opens. If misalignment fault is stored, it will appear immediately on connection.

- Note: if module shows 'Controller not found' when other modules connect fine, verify the J428 connector is fully seated and the module has power. The J428 has its own fuse — check the fuse map for ACC/Distance Sensor.

# Section 3: Live Data — Module 13 Measuring Blocks

Module 13 live data provides real-time radar sensor data including target tracking, system status, and calibration state. These channels are essential for verifying ACC is functioning correctly after calibration.

## System Status Channels

| Channel / Keyword | Description | Unit | Normal Values & Notes |
| --- | --- | --- | --- |
| ACC system status | Overall ACC operational state | — | Ready / Active / Fault / Disabled. Should show 'Ready' when system is calibrated and no faults present. |
| Calibration status | Whether J428 has been calibrated | — | Calibrated / Not calibrated / Misaligned. 'Calibrated' is the only acceptable state for normal operation. |
| Sensor operational status | Whether radar sensor is powered and operating | — | Operational / Fault. Should show Operational when module has power and no hard faults. |
| ACC active: cruise engaged | Whether ACC is currently actively controlling speed | — | Active / Inactive. Active when driver engages ACC and speed is set. |
| ACC set speed | Target cruising speed | km/h or mph | Shows the speed set by the driver when ACC is active. |
| ACC following distance setting | Driver-selected following gap | Distance bars or time | 1–5 bars on A3. Higher = larger gap. Affects how far behind the detected vehicle the car maintains. |
| System inhibit status | Reasons ACC is prevented from operating | — | Shows specific inhibit conditions: fault present, sensor blocked, speed too low, etc. |

## Radar Target Tracking Channels

| Channel / Keyword | Description | Unit | Normal Values & Notes |
| --- | --- | --- | --- |
| Target detected: yes/no | Whether radar has a tracked target (vehicle ahead) | — | Target present / No target. Should show 'No target' when road ahead is clear. |
| Target distance | Distance to tracked vehicle ahead | m | 0–150m. ACC begins active following when vehicle is within approximately 100m. |
| Target relative velocity | Speed difference between your car and target | km/h | Positive = target moving away. Negative = target approaching (ACC will brake). |
| Target lateral position | How far left or right the target is from center | m | Near 0 = target directly ahead. Offset = target partially in adjacent lane (ACC may stop tracking). |
| Target classification | What type of object is being tracked | — | Vehicle / Unknown. ACC only actively follows 'Vehicle' classified targets. |

## Calibration & Alignment Channels

| Channel / Keyword | Description | Unit | Normal Values & Notes |
| --- | --- | --- | --- |
| Horizontal alignment offset | How far the radar beam is offset from true straight ahead horizontally | ° | Within ±1.5° = calibrated and acceptable. Beyond ±2° = misalignment fault will set. |
| Vertical alignment offset | Vertical beam offset from level | ° | Should be near 0°. Large deviation = sensor physically tilted. |
| Calibration confidence | How confident the module is in its current calibration | % | Above 90% = well calibrated. Below 50% = uncertain — recalibrate. 0% = not calibrated. |
| Basic setting active | Whether a calibration routine is currently running | — | 'Not active' normally. 'Active' while running calibration Basic Settings. |
| Misalignment fault: active | Whether the hard misalignment lock is set | — | Not active = normal. Active = system locked, calibration blocked, ACC disabled. |

CRITICAL DIAGNOSTIC NOTE: The 'Misalignment fault: active' channel is the key indicator for the disconnection-caused fault. When this shows 'Active', no Basic Settings calibration routine will complete — the module rejects them. The unlock sequence in Section 4 must be performed first to clear this status before calibration is possible.

## Vehicle Dynamics Inputs to ACC

| Channel / Keyword | Description | Unit | Normal Values & Notes |
| --- | --- | --- | --- |
| Vehicle speed (from ABS) | Vehicle speed as received from ABS module | km/h | Should match instrument cluster. ACC requires accurate speed data. |
| Steering angle (from Module 44) | Steering wheel position as received by J428 | ° | ACC uses this to predict vehicle path and avoid tracking vehicles in adjacent lanes on curves. |
| Yaw rate (from Module 03) | Vehicle rotation rate as received by J428 | °/s | Combined with steering angle for vehicle path prediction. |
| Brake status | Whether driver is pressing brake | — | Pressed / Not pressed. Pressing brake disengages ACC. |
| Accelerator status | Whether driver is pressing accelerator | — | Active / Inactive. Pressing accelerator overrides ACC temporarily. |

# Section 4: Basic Settings — Complete Calibration Reference

This section is the most critical in this chapter. It covers the complete sequence for resolving misalignment faults and performing full ACC calibration. This applies directly to the situation where the J428 connector was disconnected with power on.

## Understanding Why the Fault Occurs

The J428 radar module maintains a calibration status flag in its non-volatile memory. When the module loses power unexpectedly (connector pulled with ignition on), it writes a 'misalignment detected' flag because the last thing it sensed before power loss was an abnormal signal state. This flag is not a hardware error — it is a protective software lock.

The module also maintains a separate 'calibration valid' flag. After the power-loss event, both flags are in a corrupted state: misalignment is flagged as detected, and calibration is flagged as invalid. The module will not accept new calibration until the misalignment flag is cleared, and the calibration valid flag is reset.

VCDS can clear both flags through the correct sequence of Adaptation resets followed by the Basic Settings calibration routine.

## Complete Misalignment Fault Resolution & Calibration Sequence

READ BEFORE STARTING: This procedure takes approximately 20–30 minutes. Do not rush any step. If a step fails, do not skip to the next — diagnose why the step failed before proceeding. Skipping steps in this sequence will result in calibration failure and continued ACC disable.

### Physical Prerequisites — Check These First

- J428 connector is fully seated with audible click — push firmly until locked

- J428 connector pins are straight, not bent or corroded — inspect visually

- Bumper cover is fully reinstalled and clipped — not pressing against the sensor

- Sensor face has no cracks, moisture, or physical damage

- Verify the J428 fuse is intact — check fuse map in owner's manual for ACC fuse location

- No physical damage to J428 housing or mounting bracket

STOP HERE IF: The J428 connector pins are bent, the sensor face is cracked, or the sensor mounting bracket is bent. Physical repair must precede any software calibration. A physically damaged or misaligned sensor cannot be successfully calibrated.

### Step 1: Connect and Read All Faults

- Engine running. VCDS connected. Open Module 13.

- Click [Fault Codes]. Record every code present. The most common codes after a disconnection event are:

- 01316 — Radar sensor: misalignment

- 01566 — Radar sensor: not calibrated

- C101300 — Distance sensor: misalignment

- U codes — communication faults from other modules (usually secondary)

- Screenshot or write down all codes before proceeding.

### Step 2: Clear Fault Codes

- With Module 13 still open: click [Fault Codes] → [Clear Codes] → confirm.

- Wait 15 seconds.

- Click [Fault Codes] again to re-read.

- If codes return immediately (within 15 seconds of clearing): the module is actively re-generating them. This is expected — proceed anyway. The goal at this step is not to permanently clear codes but to reset the fault counter state.

- If codes do NOT return immediately: move to Step 3.

### Step 3: Adaptation Reset — Sensor Operational State

This is the critical step that most diagnostic guides miss. You must reset the adaptation channels that store the misalignment flag before calibration will be accepted.

- Module 13 open. Click [Adaptation].

- In the search box, type: 'sensor' — filter the dropdown.

- Look for any of these channels (name varies slightly by VCDS version):

- 'Sensor operational state'

- 'Alignment status'

- 'Sensor calibration status'

- 'Radar sensor: initialization'

- 'ACC sensor: factory reset'

- When found: read the current value. It will show a non-zero or fault state value.

- Change the value to 0 (zero) or to the 'reset' / 'factory' option shown in the dropdown.

- Click [Do It!] to save.

- If VCDS asks for security access before this adaptation: the code is NOT required for Module 13 calibration on most A3 variants — if prompted, try proceeding without it first.

NOTE: If you cannot find any 'sensor operational state' or 'calibration status' channel in Module 13 adaptations on your specific car, proceed directly to Step 4. Some firmware versions handle this reset internally during the Basic Settings routine itself.

### Step 4: Power Cycle the J428 Module

After resetting the adaptation channels, the module needs a full power cycle to apply the reset state before calibration.

- Turn ignition completely OFF.

- Wait 60 full seconds — not 10, not 30. 60 seconds allows all module capacitors to fully discharge.

- Do NOT disconnect the HEX-V2 during this wait.

- Turn ignition back ON. Start engine.

- Wait 30 seconds for all modules to fully initialize before opening Module 13 again.

### Step 5: Re-read Faults After Power Cycle

- Open Module 13.

- Click [Fault Codes].

- Observe: are the misalignment codes still present? If yes — clear them again and check whether they return in less than 10 seconds.

- If codes return in less than 10 seconds still: the adaptation reset in Step 3 may not have saved correctly. Return to Step 3 and verify the adaptation value was saved (re-read the channel after saving to confirm).

- If codes are gone or take more than 30 seconds to return: proceed to Step 6.

### Step 6: Physical Preparation for Calibration

The J428 calibration routine has strict physical requirements. If these are not met, the routine will fail or produce an incorrect calibration that will re-fault within one drive cycle.

- Find a flat, level surface — a parking lot or road with no crown is ideal. Avoid hills, crowned roads, or garage ramps.

- Vehicle must be pointing straight — front wheels centered, not turned.

- Clear area directly ahead of the vehicle for at least 10 meters — no other vehicles, walls, or large objects in the radar field.

- Engine running, vehicle stationary.

- All calibration aids must be removed from bumper — no protective film over radar area, no mud, snow, or ice on the bumper face where the sensor is located.

- If the front bumper cover was recently repainted: paint should be fully cured (at least 24 hours).

WARNING: Running the calibration routine while on a crowned road, while the vehicle is angled, or with obstructions in front of the radar will produce an incorrect calibration offset. The sensor will be calibrated to point off-center and will re-fault within 50–100 miles of driving as the module detects the sustained misalignment during normal operation.

### Step 7: Run the Calibration Basic Settings Routine

- Module 13 open. Engine running. Physical prerequisites confirmed from Step 6.

- Click [Basic Settings].

- From the dropdown menu, look for one of these routines (name varies by VCDS version):

- 'ACC distance sensor: calibration'

- 'Radar sensor calibration'

- 'Sensor alignment: automatic'

- 'Basic setting — ACC sensor'

- 'Factory reset and calibration'

- Select the calibration routine.

- Click [Do It!] or [On/Off/Next].

- VCDS shows 'Basic Setting: On' or 'ADP RUN'. The routine is running.

- The routine takes 15–60 seconds. During this time the module reads the radar return pattern and calculates the alignment offset relative to vehicle center.

- VCDS shows 'ADP OK' or 'Basic Setting Complete' when successful.

- If VCDS shows 'ADP ERROR': see troubleshooting section below.

### Step 8: Clear All Faults Post-Calibration

- After calibration completes: click [Fault Codes] → [Clear Codes].

- Wait 30 seconds.

- Re-read fault codes. The misalignment and not-calibrated codes should now be absent.

- Check live data: 'Calibration status' should now show 'Calibrated'. 'Misalignment fault' should show 'Not active'.

### Step 9: Verification Drive

- Drive at 40+ mph on a straight road.

- Engage ACC by pressing the cruise control stalk Set button.

- ACC should activate and hold speed.

- Drive behind another vehicle — ACC should automatically reduce speed to maintain following distance.

- After 10 minutes of driving: return to VCDS and re-read Module 13 fault codes.

- No new faults = calibration successful and stable.

- If misalignment code returns after driving: the calibration offset stored is incorrect. Repeat from Step 6 with more attention to physical setup.

## Calibration Routine: Troubleshooting

| Problem | Solution |
| --- | --- |
| Routine quits immediately ('ADP ERROR' instantly) | Misalignment fault still active. Return to Step 3 — adaptation reset did not save. Re-verify the adaptation channel value reads 0 after saving, then repeat power cycle. |
| Routine runs but times out without completing | Physical obstruction in radar field — another vehicle, wall, or large object is within 10m directly ahead. Clear the area and retry. |
| Routine completes but misalignment code returns within 5 minutes of driving | Calibration offset is wrong — vehicle was not on level ground or was not pointing straight during calibration. Repeat from Step 6 on better surface. |
| Module 13 does not connect at all | J428 fuse blown, connector not fully seated, or J428 module has failed. Check fuse first, then verify connector, then test with a known-good J428 unit. |
| Adaptation channels not visible in Module 13 | Specific VCDS version may not expose all UDS adaptation channels for J428. Update VCDS to latest version. If still missing, proceed directly to Basic Settings calibration — some versions handle the reset internally. |
| VCDS shows 'Basic Settings not available' | Active fault codes are blocking Basic Settings. Clear ALL fault codes from Module 13, then immediately attempt Basic Settings before the module re-logs them. You may have a 10–15 second window. |

# Section 5: Adaptation Channels — Module 13

Module 13 adaptation channels on the J428 are limited compared to other modules — the module is purpose-built and most of its behavior is fixed in firmware. The following channels are accessible via VCDS.

| Channel Keyword | Function | Notes |
| --- | --- | --- |
| Sensor operational state / alignment status | Flag indicating sensor calibration and misalignment state | Core channel for misalignment fault resolution. Set to 0/factory reset as part of calibration unlock sequence. |
| Calibration status | Whether sensor is currently calibrated or needs calibration | Read-only in normal operation. Changes to 'calibrated' after successful Basic Settings routine. |
| Horizontal alignment offset: stored value | The actual stored calibration offset for horizontal beam direction | Should be within ±1.5°. Read after calibration to confirm value is reasonable. Values outside ±3° indicate calibration surface was not level or vehicle was angled. |
| ACC: available following distance settings | Which following distance bars/levels are presented to the driver | 1–5 levels. Factory setting. Can restrict to fewer levels if desired. |
| Front Assist: warning timing | How early the collision warning activates before potential impact | Early / Normal / Late. Factory: Normal. 'Early' provides maximum warning time at expense of more frequent false warnings. |
| Autonomous emergency braking: active | Whether AEB will automatically apply brakes | Active / Inactive. Leave Active — this is a safety function. |

# Section 6: Module 13 Coding

Module 13 coding is minimal — the J428 is a purpose-built sensor with little configurable behavior at the coding level. Most configuration is done through adaptations and Basic Settings.

| Byte | Bit/Value | Function | Notes |
| --- | --- | --- | --- |
| Byte 0 | Bits 0–7 | Sensor hardware variant identification | Factory coded. Do not modify. Identifies which J428 variant is installed. |
| Byte 1 | Bit 0 | ACC: active speed control available | 1=ACC present (adaptive cruise). 0=cruise control only (no speed following). Factory 1 on A3 with ACC. |
| Byte 1 | Bit 1 | Front Assist: installed | 1=collision warning system active. Factory 1. |
| Byte 1 | Bit 2 | AEB: autonomous emergency braking installed | 1=auto braking capable. Match to hardware. Not all A3 variants include AEB. |
| Byte 2 | Bit 0 | Traffic Jam Assist: installed | 1=stop-and-go ACC available (full stop and restart). 0=ACC only active above 20 km/h. |

# Section 7: Fault Code Reference — Module 13

Module 13 fault codes are primarily focused on sensor calibration state, electrical supply, and communication with other modules that provide vehicle dynamic data.

## Calibration & Alignment Codes

| Code | Description | Causes & Resolution |
| --- | --- | --- |
| 01316 | Radar Sensor: Misalignment Detected | Primary fault after disconnection event. Sensor calibration offset is corrupted or sensor physically misaligned. Follow complete calibration sequence in Section 4. If fault returns after correct calibration: sensor may be physically damaged or mounting bracket bent. |
| 01566 | Radar Sensor: Not Calibrated | Sensor has never been calibrated or calibration was lost. Run Basic Settings calibration routine. This code alone (without 01316) is simpler to resolve — only calibration needed, no adaptation reset required first. |
| C101300 | Distance Sensor: Misalignment | Extended fault code format for the same misalignment condition as 01316. Treated identically. |
| C101400 | Distance Sensor: Calibration Required | Extended format for not-calibrated condition. Same as 01566. |
| C101500 | Radar Sensor: Sensor Blocked | Radar beam path obstructed — mud, ice, snow, or damage to bumper cover directly over sensor. Clean the area and retry. If code persists with clean sensor area: bumper foam/plastic may be deformed and blocking the beam internally. |

## Power Supply & Hardware Codes

| Code | Description | Causes & Resolution |
| --- | --- | --- |
| 01577 | Radar Sensor: Supply Voltage Fault | J428 supply voltage too low or too high. Check the ACC fuse. Check battery voltage during engine running (should be 13.8–14.4V). Poor ground at J428 chassis ground point can cause this. |
| 01578 | Radar Sensor: Internal Fault | J428 module internal hardware failure. Cannot be resolved by calibration. Module replacement required. |
| 01579 | Radar Sensor: Temperature Fault | Module temperature out of operating range. Typically resolves itself once module cools or warms. If persistent: module failure. |
| 00920 | Radar Sensor — No Signal | No radar return signal detected. Sensor face blocked, or sensor hardware failed. Clean sensor area first. |

## Communication Fault Codes

| Code | Description | Causes & Resolution |
| --- | --- | --- |
| U111300 | Communication with Engine Control Module Lost | J428 cannot receive data from Module 01. CAN wiring issue or Module 01 has failed. Run AutoScan to see if Module 01 has corresponding fault. |
| U111400 | Communication with ABS Module Lost | J428 cannot receive wheel speed and vehicle dynamics data from Module 03. ACC requires this data to function. Check Module 03 independently. |
| U111600 | Communication with Transmission Module Lost | J428 cannot send gear selection commands to Module 02. Check Module 02 for faults. |
| U111700 | Communication with Steering Module Lost | J428 cannot receive steering angle data. Check Module 44 for faults. |

NOTE: Communication faults (U codes) in Module 13 are almost always secondary — they result from a fault in another module preventing data transmission. Always run a full AutoScan and address faults in the primary modules (01, 02, 03, 44) before troubleshooting Module 13 U codes. Fixing the upstream fault usually clears the Module 13 U codes automatically.

## Front Assist Fault Codes

| Code | Description | Causes & Resolution |
| --- | --- | --- |
| C100900 | Front Assist: System Fault | Front Assist (pre-collision warning) system fault. Often secondary to radar calibration fault. Resolve radar calibration first. |
| C100100 | Front Assist: Camera Signal Implausible | On A3 variants with combined radar+camera ADAS: camera signal not correlating with radar. Check camera module (Module 0E) separately. |
| C100200 | AEB: Autonomous Braking Disabled | AEB has been disabled, either by coding, adaptation, or fault. Check AEB adaptation channel status. |

# Section 8: Live Data During ACC Operation — What to Expect

The following describes what you should see in Module 13 live data during normal ACC operation. Use this as a reference to verify the system is working correctly after calibration.

## At Highway Speed with ACC Engaged — Normal Readings

| Channel | Expected Value |
| --- | --- |
| ACC system status | Active |
| Calibration status | Calibrated |
| Misalignment fault | Not active |
| Calibration confidence | >90% |
| ACC set speed | Your chosen cruise speed in km/h or mph |
| Target detected | No target (if road clear ahead) |
| Vehicle speed from ABS | Matching your actual vehicle speed |
| Steering angle | Near 0° on straight road |

## When ACC is Actively Following a Vehicle Ahead — Normal Readings

| Channel | Expected Value |
| --- | --- |
| Target detected | Target present |
| Target distance | Varies based on speed and following distance setting — typically 30–80m at highway speed |
| Target relative velocity | Near 0 km/h (matching speed of vehicle ahead) — should not be persistently positive or negative |
| Target lateral position | Near 0m — target is directly ahead. Values >1.5m mean target is in adjacent lane and ACC may stop tracking. |
| Target classification | Vehicle |
| ACC system status | Active (following) |

# Section 9: Quick Reference — Module 13

## Calibration Checklist — Fast Reference

| Step | Action |
| --- | --- |
| 1 | Verify J428 connector fully seated and pins intact |
| 2 | Verify bumper cover fully installed, no obstructions on sensor face |
| 3 | Open Module 13 — read and record all fault codes |
| 4 | Clear all fault codes |
| 5 | Adaptation → find 'sensor operational state' or 'alignment status' → set to 0/factory reset → save |
| 6 | Ignition OFF — wait 60 seconds full — ignition ON — start engine — wait 30 seconds |
| 7 | Open Module 13 — clear any codes that returned |
| 8 | Position car: flat level surface, pointing straight, 10m clear ahead |
| 9 | Basic Settings → select ACC/radar calibration routine → Do It! → wait for ADP OK |
| 10 | Fault Codes → Clear All → verify no immediate return of misalignment codes |
| 11 | Live Data → confirm: Calibration Status = Calibrated, Misalignment Fault = Not Active |
| 12 | Test drive 10+ miles — re-read codes — verify clean |

## Common Fault Codes — Fast Lookup

| Code | Description | Quick Action |
| --- | --- | --- |
| 01316 | Misalignment detected | Full calibration sequence — Steps 1–12 above |
| 01566 | Not calibrated | Basic Settings calibration only — Steps 6–12 |
| C101300 | Misalignment (extended format) | Same as 01316 |
| C101500 | Sensor blocked | Clean sensor area, retry calibration |
| 01577 | Supply voltage fault | Check ACC fuse, check charging voltage |
| 01578 | Internal fault | Module replacement required |
| U1113/1114/1116/1117 | CAN communication lost | Fix primary module faults first |

— End of Module 13: Distance Regulation / ACC —

Continue to Module 17: Instrument Cluster